Title: Optimum Miscible CO2-SWAG/WAG Injection in the Bakken Formation

Yongan (Peter) Gu

Petroleum Systems Engineering, Faculty of Engineering and Applied Science

University of Regina, Regina, Saskatchewan S4S 0A2, CANADA

Abstract:

The Bakken formation has approximately 271-503 billion barrels of light oil in place and becomes a vast and vital light oil resource in Western Canada and the adjacent states in USA. This tight oil formation has permeabilities of 0.01 to 1 mD and the oil recovery factor of its primary production is estimated to be 1-3%. Water is very difficult to inject in large volumes into it, whereas CO2 may breakthrough it more easily than water. Thus how to properly inject water and/or CO2 and maximize the oil recovery in the Bakken formation becomes a key technical question.

In this paper, miscible CO2 simultaneous water-and-gas (CO2-SWAG) injection and CO2 water-alternating-gas (CO2-WAG) injection in the Bakken formation are experimentally studied. A total of fifteen coreflood tests are conducted by applying waterflooding, miscible CO2 flooding, miscible CO2-WAG and CO2-SWAG injection. The miscible CO2-SWAG injection with an injected water‒gas ratio (WGR) of 1:3 in volume achieves the highest oil recovery factor and carbon utilization factor, which is followed by the miscible CO2-WAG injection with the optimum slug ratio of approximately 1:1, miscible CO2 flooding, and waterflooding. The WGR and the WAG slug size show strong effects on the fluid production trends in the miscible CO2-SWAG and CO2-WAG injection processes, respectively. In comparison with water or CO2 alone, the water‒CO2 mixture has a much lower mobility in the tight formation. Hence, the high oil recovery in the optimum CO2-SWAG/WAG injection is attributed to a well-controlled water‒CO2 mobility and a substantially weakened waterblocking effect.

Biography:

Dr. Yongan (Peter) Gu obtained his Ph.D. degree at the University of Alberta (Canada) in 1999. He was hired as an assistant professor in 1999, an associate Professor in 2002, and a full professor in 2006 at the University of Regina (Canada). His three major research areas are CO2-enhanced oil recovery (CO2-EOR), carbon capture utilization and storage (CCUS), solvent- and thermal-based EOR in post-CHOPS reservoirs. Dr. Gu has conducted 34 large research projects funded by the Canadian governments and the petroleum industry. He has co-authored over 80 refereed journal papers, 4 invited review articles, and presented 54 peer-reviewed conference papers.

Presenting author details: 
Full name: Dr. Yongan (Peter) Gu
Contact number: (306) 585-4630, Peter.Gu@Uregina.Ca
Twitter account: N/A
Linked In account: N/A

Session name/ number: Track 8-3
Category: Oral presentation