# Modeling of Yb: YAG Inoslab Laser Amplifier

Mohanna Najafi, INLC, IRI

Arezue Sepehr, INLC, IRI

Ehsan Tanhaei, INLC, IRI

Mohammad Mahdi Majid of, INLC, IRI

Seyedhasan Nabavi, INLC, IRI

### Abstract

### Investigation of multipass Yb:YAG Innoslab amplifier has been presented In this paper. An Innoslab amplifier consists of a longitudinally, partially pumped slab crystal. Due to this structure high pump intensity as well as a high quality beam profile obtains from uniform pumped volume. Owing to decrease the gain medium doping level a crystal with larger scale could be utilized. This leads to increase the single pass absorption besides outstanding thermal management due to vast cooling surfaces in both faces.

### In this paper thermal effect analyses and slab temperature distribution are defined by FEA method in the LASCAD commercial software. Innoslab pumping architecture provides a homogeneous pump volume in Yb:YAG crystal which results one dimensional heat flow. Cylindrical thermal lens and stress distribution is determined by solving heat equation regarding mentioned boundary condition.

### Moreover spatial rate equations along main axis are solved for contra-propagating pumping and one pass laser through amplified medium. Therefore a couple of laser features such as gain and absorption coefficient distribution and absorbed power distribution could be precisely calculated. Amplified output power graph Vs pump power in both edges and regarding various doping levels is depicted in fig. 1.

### Image

Recent Publications (minimum 5)

P. Russbueldt,D. Hoffmann, M. Hofer, et al,”Innoslab Amplifiers”,Quantum Elec. , Vol.21, No. 1, pp. 966-971, 2015.

P. Russbueldt, T. Mans, G.Rotarius, et al, “400 W Yb:YAGInnoslabfs-amplifier”, Optics Express, Vol.17, No. 15, pp. 12230-12245, 2009.

P. Russbueldt, T. Mans, J. Weitenberg, D. Hoffmann, and R. Poprawe, " compact diode pumped 1.1 Kw Yb:YAG Innoslab femtosecond amplifier ", Optics Letters, Vol. 35, No. 24, 2010.

Gilbert L. Bourdet,“Theoritical investigation of quasi-three- level longitudinally pumped continuous wave lasers”,Applied Optics, Vol. 31, No. 6, pp. 966-971, 2000.

Biography

Arezue Sepehr recived her MS. degree in physics from Bu. Ali Sina university (BAS), Iran. From 2007 to 2017, she has been  researcher in Iranian National Center for Laser Science and technology. Her research interests primarily are in the field of CW and short pulse solid state laser system. She also conducts numerical simulation in condense-matter physics. Presently, her research is concentrated on Slab laser, phase coupling of diode laser and characterization of laser beam and optical component.

Email:arezue.sepehr@gmail.com

Notes/Comments: