Cadmium (II) removal from an aqueous solution using CSCMQ Copolymer

Prapti U. Shaha*, Nirav P. Ravala, Nisha K. Shahb,

a*Department of Environmental Science, School of Sciences, Gujarat University,

Ahmedabad, Gujarat - 380 009

bDepartment of Chemistry, School of Sciences, Gujarat University,

Ahmedabad, Gujarat - 380 009

Abstract

In this work, an approach was developed to synthesize copolymer by grafting 5-Chloromethyl-8-Hydroxyquinoline (CMQ) onto Cassava starch (CS) for the removal of Cd(II). Characterization experiments were done by using Scanning Electron Microscopy (SEM) and Fourier Transform Infrared (FTIR) analysis. The study was performed under different experimental conditions of initial metal ion concentration, adsorbent dose, time, pH and temperature. Batch adsorption experiment showed that grafted copolymer had strong adsorption capacity of 51.546 mg/g, 53.191 mg/g and 55.249 mg/g at 298 K, 308 K and 318 K for Cd(II) removal. The kinetics data were analyzed using pseudo-first-order, pseudo-second-order and intraparticle diffusion models. The pseudo-second-order model best described the adsorption of Cadmium (II) ions onto copolymer. Langmuir model fitted well and thermodynamics parameters were analyzed where results revealed that the adsorption process was a spontaneous and endothermic in nature.

Key–Words: Cassava Starch; Copolymer; Cadmium (II); Adsorbate; Adsorbent.

Ms. Prapti U. Shah (*Corresponding and presenting author)

Research Scholar

Department of Environmental Science,

School of Sciences,

Gujarat University,

Ahmedabad – 380 009

Gujarat, INDIA.

E-mail: prapti.shah1126@gmail.com

(M): +91 9558825191

Mr. Nirav P. Raval 					Dr. N. K. Shah

Research Scholar 					Professor

Department of Environmental Science 			Department of Chemistry

School of Sciences 					School of Sciences

Gujarat University 					Gujarat University

Ahmedabad – 380 009					Ahmedabad – 380 009

Gujarat, INDIA. 						Gujarat, INDIA.

Email: nirav.raval90@gmail.com				E-mail: nishchem.2004@gmail.com