Analysis of physico-chemical parameters of groundwater located near Arabian Sea tidal water, district Thatta, sindh Pakistan.

G. M. Mastoi1 and  M. Amin Qureshi2

1 Institute of advanced research in chemical sciences university of Sindh, Jamshoro Pakistan

2  Institute of environmental sciences, Sindh Madressatul Islam University, Karachi, Sindh Pakistan

Abstract

The main objective quality the current study is to focus on the influence of tidal activities of Arabian Sea water on groundwater quality of taluka Keti    Bandar, Thatta sindh Pakistan.For the assessment of physico-chemical parameters ten (10) sampling stations were selected to cover the different areas of Keti Bandar Taluka. Sampling was carried out on 22 February 2017. Water quality parameters such as temperature, dissolved oxygen, conductivity, salinity, pH, total dissolved solids (TDS) and GPS were recorded on the spot. Eco-toxic elements ions (Cd+2, Cu+2, Fe+2, Mn+2, Ni+2, Pb+2 and Zn+2) levels were determine by flame atomic absorption spectrometer (FAAS) after pre-concentration. Heavy metals (Na, Ka, Ka, Mg) levels were estimated by FASS after appropriate dilation.   Residues (total, volatile and fixed) were determined after evaporation of water samples on hot plate fallowed by drying at 105 0C in an oven. The remaining residue was burnt in a muffle furnace at 550 0C for one hour. The fixed residue was calculated after reduction of volatile residue from the fixed residue chlorides, m-alkalinity as Ca CO3 ions and their and hardness as Ca CO3 were determine tritramatery. Nitrate content in water sample was estimated by brucine method using spectrophotometer. The obtained results were compared with the standards a established by WHO, European standards and Sindh Environmental Protection Agency. Some impermissible values of few parameters were recorded and discussed in detail.

Keywords: influence of sea tidal water, groundwater quality, physico-chemical parameters, K.T Bandar, Thatta, Sindh.