# Technology transferring and study on dyed removal efficiency from small textile enterprise wastewater by alum coagulation followed with burning rice-husk adsorption

Nanthanat Sriprasert, PhD, Mahasarakham University, Thailand

Pakpong Sriprasert, MEng, Mahasarakham University Thailand

### Abstract

### This research was aimed to study the decolorization efficiency of dyed wastewater from small and medium textile enterprise groups in Maha Sarakham, Thailand. The selected technology was alum coagulation and flocculation followed by burning rice-husk adsorption. The major advantages of this method were due to its relative high dye removal efficiency and the simplicity of implementation. In addition, the required chemical reagents as well as equipment are inexpensive and could be provided locally as burning rice-husk was a residual material/by product from rice-husk mill plant which was easily affordable. The effects of initial pH adjustment (3-12) and alum dosage (1-10 g alum/L) on dye removal efficiency were tested with the real dyed wastewater in coagulation process. Thereafter, the effectiveness of adsorption process for removing dye was further assessed with 5-25 g burning rice husk/L under adjusted pH of 2-8.

### The characteristics of real dyed wastewater had an initial pH of 8.6, turbidity of 124.3 NTU, absorbance for 12.5-fold dilution at 496 nm was 1.77 and COD of 1,676 mg/L. Results showed that optimum coagulant (alum) dosage was 8 g/L at adjusted initial pH of 11. This stage achieved 74.8, 93.2 and 95.3 % for turbidity, color and COD removal efficiency, respectively. Coagulated wastewater was further treated by the burning rice-husk adsorption in order to remove residual contaminants. For adsorption step, the optimum burning rice-husk amount was 10 g/L at initial adjusted pH of 10 with equilibrium time of 20 minutes which gave 98.1, 96.9 and 75.3 % resulted in the overall removal efficiency of 99.7, 94.2 and 94.6 % for turbidity, color and COD, respectively. In the section of technology transfer training about “Technology transferring and study on dyed removal efficiency from small textile enterprise wastewater by alum coagulation followed with burning rice-husk adsorption”, trainees concluded that the training program were satisfied and benefitted.

Recent Publications

- Zhang Y., He S., Sriprasert N., Banks C.J. and Heaven S., 2017, Competing demands for trace elements in anaerobic digesters, Paper presented to the 1st International Congress on Metals in Anaerobic Biotechnologies (IMAB17), Seville, Spain, the 4th - 6th October. Page 33 - 34.

- Sriprasert N. and Sriprasert P., 2018, Hydrazine reduction process for copper recovery from spent copper etching solution from printed circuit board manufacturing, E-poster presented to the 9th World Convention on Recycling and Waste Management, Osaka, Japan, the 22nd-23rd October. Page 34.

- Sriprasert N. and Sriprasert P., 2018, Recovery of copper from spent copper etching solution using wasted aluminum drill entry sheets from PCBs industry, E-poster presented to the 5th Global Summit and Expo On Pollution Control, Prague, Czech Republic, the 25th – 27th October.

- Sriprasert N. and Sriprasert P., 2018, Using of iron filings for copper recovery from spent copper etching solution by cementation process, E-poster presented to the 9th International Conference on Recycling: Reduce , Reuse and Recycle, Vancouver, British Columbia, Canada, the 5th – 6th December.

- Sriprasert P. and Sriprasert N., 2018, Electrowinning process for copper removal from spent etching solution in printed circuit boards manufacturing, E-poster presented to the 9th International Conference on Recycling: Reduce , Reuse and Recycle, Vancouver, British Columbia, Canada, the 5th – 6th December.

Biography

Dr Nanthanat Sriprasert and Pakpong Sriprasert are lecturers in Faculty of Environment and Resource Studies at Mahasarakham University, Thailand. Their research has currently focused on pollution control and environmental sustainability, especially, the process of water pollution and treatment of both domestic and industrial wastewater. The areas of their expertise are wastewater treatment technologies including chemical and biological processes. They have built valuable results after years of experience in these researches. Dr Sriprasert, the corresponding author, obtained a BEng and MEng in Environmental Engineering from Suranaree University of Technology, Thailand. She was awarded her PhD by University of Southampton, the UK.

Email: nanthanat@msu.ac.th

Notes/Comments:

Presenting author details 
Full name: Nanthanat Sriprasert
Contact number: +66639954566
Twitter account: -
Linked In account: -

Session name/ number: Track 5 Water Pollution and Treatment:
Category: (Oral presentation/ Poster presentation): E-poster