EFFECTS OF USING TANNERY SLUDGE ON SOIL PROPERTIES AND HEAVY METALS CONCENTRATION IN SOME VEGETABLES GROWN IN AMENDED SOILS

Mahadi Garba Doka

Department of Chemistry, Federal College of Education (Tech), Bichi

Email: mahdgar@gmail.com

Tannery sludge waste is currently being used by many farmers for cultivation of vegetables on large scale for human consumption. The study investigated the effect of tannery sludge on some soil properties and levels of the metals in Amaranthus hybridus and Corchorus olitorius grown on the tannery amended soil. Heavy metals were analyzed in the harvested vegetable tissues using Atomic Absorption Spectrophotometer. The result showed the soil pH, organic matter content and conductivity of the amended soils increased significantly (P < 0.05) compared to the controls. It also revealed higher concentration ranges (13.80 – 17.50mg/kg) of nickel in the leave, lead (50.10 – 83.80mg/kg) and Cadmium (1.00 – 3.80mg/kg) in the root of Corchorus olitorius. While higher concentration of chromium (1453.50mg/kg) was observed in Amaranthus hybridus root. The concentrations, except nickel in the edible tissues (stem and leave) were above the FAO/WHO recommended safe limits. The Transfer Factor for all the metals except Ni, revealed the enormous potential of the vegetables for phytoextraction. Hence, stringent guidelines set for tannery sludge applications on agricultural land should be totally enforced.

Key words: Heavy metals, Pollution, Sludge, Tannery.