# Emission source of PAHs in PM2.5 particle in ambient air of Sakai City, Osaka

Oanh Pham, Master, Osaka Prefecture University, Japan

Norimichi Takenaka, Prof., Osaka Prefecture University, Japan

### Abstract

Statement of the Problem: PM2.5 whose diameter is smaller than 2.5 m can travel through airway and deposit into lung tissue. Moreover, PM2.5 carries many harmful compounds including polycyclic aromatic hydrocarbons (PAHs), byproduct from incomplete combustion of organic materials. For better knowledge about PM2.5-bound PAHs (p-PAHs) in Sakai City, Osaka, we measured PAHs in gas phase and particle phase (PM2.5) and apportion the sources as well as assess the lifetime lung cancer risk (LLCR). Methodology & Theoretical Orientation: Using the low volume air sampler (LVAS) with flowrate of 4 L min-1, particles and gas phase of PAHs were collected on glass fiber filter (GFF) and polyurethane foam (PUF), respectively. Samples were collected on the rooftop of 3-storey building in Osaka Prefecture University, Sakai City, Osaka, Japan. HPLC was used for analyzing 11 PAHs including Phenanthrene, Anthracene, Fluoranthene, Pyrene, Benz(a)Anthracene, Chrysene, Benzo(b)Fluoranthene, Benzo(k)Fluoranthrene, Benzo(a)Pyrene, Dibenz(a,h)Anthracene, Indeno(1,2,3-cd)Pyrene. Diagnostic ratios (DR) method for apportioning the PAH’s sources. Backward trajectory was used for tracing source location. Findings: Main sources of PAHs were gasoline evaporation, vehicular emission, and coal combustion. There was no significant correlation between PAHs and air pollutants (NOx, SO2, NMHC, etc.) at nighttime. The results showed that the gaseous PAHs were from local sources while the PM2.5-bound PAHs were from distant place. Backward trajectory results showed East Asia might be one of emission source’s location of PAHs in PM2.5 in Sakai City, Japan. The mean TEQ and MEQ in particle phase values were higher than annually averaged standard of 1 ng/m3 of EPA. Conclusion & Significance: Based on the TEQ values, Sakai City air were considered to be harmful to human health. Because atmospheric pollution is a global issue, there is a need for international policy about PM2.5 and harmful substances on it.

### Image

Backward trajectory

Figure 1. Apportion main source and locate the main emission source of p-PAHs using Diagnostic ratio (DR) method and Backward trajectory.

Recent Publications

- Hien T. T., Oanh P. (2017) Occurrence of Bisphenol A in the atmospheric aerosols in Ho Chi Minh City, Vietnam during 2012-2014. The 5th analytica Vietnam Conference 2017:107-114.

- Oanh P. (2016) Measurement of particle and gaseous PAHs in ambient air by low volume air sampler. The 57th Annual Meeting of Japan Society for Atmospheric Environment.

- Oanh P. (2017) Master thesis: Measurement of PAHs in PM2.5 particle and gaseous phase in the ambient air of Sakai City by low volume air sampler. Osaka Prefecture University.

Biography

Oanh Pham is a doctoral student who is interested in organic pollution in atmosphere. After getting Bachelor degree in Environmental Technology, she joined master course of Applied Chemistry Department, Major Environmental Chemistry. Her main object is organic contaminants, especially polycyclic aromatic hydrocarbons (PAHs). She has experienced collecting TSP and PM2.5, analyzing BPA and PAHs along with using receptor models for acknowledging the pollution trends, sources and effects. Interested in the effects of PAHs, besides using TEF method to assess the cancer risk, she had spent time in a biology laboratory to learn about Ames test to test the mutagenic of chemical. Currently, she has investigated the reactions between PAHs and air pollutants in ice under effect of light.

Email: mz105098@edu.osakafu-u.ac.jp

Notes/Comment: