A Comparative Pyrolysis of Microalgae Botryococcus Braunii, Spirulina Sp, and Chlorella Sp

Nickolas Chen, Oxbridge Academy of the Palm Beaches, USA

Teresa. Thornton, PhD, Oxbridge Academy of the Palm Beaches, USA

Karen Hinkley, PhD, Oxbridge Academy of the Palm Beaches, USA

### Abstract (300 word limit)

The microalgae, Botryococcus braunii, Chlorella sp., and Spirulina sp., have been reprised as future sources of biofuel due to its concentrations of hydrocarbons and potential for oil production. The three algae underwent pyrolysis in a fixed-bed reactor, a process in which the algae were thermally decomposed, and then oil quantities were compared.  The fuel quality of each different algae yield may vary depending on both the biochemical compositions of the strains as well as their response to the pyrolysis reaction.

Chlorella sp. and Spirulina sp. powders were obtained from a commercial source whereas Botryococcus braunii samples were grown using a modified Bristol Medium at a concentration of 0.02%, room temperature, and humidity, 350 foot-candle light intensity, and a photoperiod of 16/8h light/dark cycle. The Botryococcus braunii feedstock was cultured, harvested, and dried at 60C for 24h to produce a powdered feedstock.

Preliminary results from the pilot study indicated that 5mg of Chlorella sp. can produce approximately 2mg of biofuel. However, the other two algal samples were too small to record an accurate result. Presently a second study is in place to more accurately compare the biofuel yields.  Thirty 5g samples of each algae will be pyrolyzed and run through an HPLC to determine precise yields for biofuel. Since Spirulina sp. is 8% fat (USDA, 2016), Chlorella sp. is 22.5% fat and Botryococcus brannii is 34.9% fat (GH et al, 2014), it is expected that the Botryococcus braunii will produce the greatest amount of useable biofuel.

Key Words: Botryococcus Braunii, Chlorella sp., Spirulina sp., Biomass, Pyrolysis Products, HPLC Analysis

Recent Publications (minimum 5)

- Anand, V., Sunjeev, V., & Vinu, R. (2016). Catalytic fast pyrolysis of Arthrospira platensis (spirulina) algae using zeolites. Journal of 
Analytical and Applied Pyrolysis, 118, 298-307. doi:10.1016/j.jaap.2016.02.013

- Banerjee, A., Sharma, R., Chisti, Y., & Banerjee, U. C. (2002). Botryococcus braunii: A Renewable Source of Hydrocarbons and Other Chemicals. Critical Reviews in Biotechnology, 22(3), 245-279. doi:10.1080/07388550290789513

- Chen, C., Cheng, Z., & Chen, F. (2017). Experimental Study on Microwave Pyrolysis Characteristics of Microalgae Chlorella Vulgaris. Proceedings of the 2nd 2016 International Conference on Sustainable Development (ICSD 2016). doi:10.2991/icsd-16.2017.121

- Peng, W. (2001). Pyrolytic characteristics of microalgae as renewable energy source determined by thermogravimetric analysis. Bioresource Technology, 80(1), 1-7. doi:10.1016/s0960-8524(01)00072-4

- Samori, C. & Torri, C. (2013). Alternative Methods for Extraction of Hydrocarbons from Botryocococcus Braunii. In James Weifu Lee (ed.), Advanced Biofuels and Bioproducts, Springer, New York, 651-670.

- Vassilev, S. V., & Vassileva, C. G. (2016). Composition, properties and challenges of algae biomass for biofuel application: An overview. Fuel, 181, 1-33. doi:10.1016/j.fuel.2016.04.106

- Yang, X., Wang, X., Zhao, B., & Li, Y. (2014). Simulation Model of Pyrolysis Biofuel Yield Based on Algal Components and Pyrolysis Kinetics. BioEnergy Research, 7(4), 1293-1304. doi:10.1007/s12155-014-9467-z

- Yanik, J., Stahl, R., Troeger, N., & Sinag, A. (2013). Pyrolysis of algal biomass. Journal of Analytical and Applied Pyrolysis, 103, 134-141. doi:10.1016/j.jaap.2012.08.016

Biography (150 word limit)

Nickolas Chen is a high school senior student who is active in his local scientific community. He has developed a passion for STEM subjects and often volunteers either peer tutoring or helping out at a science center. Nickolas is highly interested in renewable fuels and has dedicated much of his other time towards biofuel research. Nickolas has been culturing and experimenting on algae in order to learn more about the intricacies of Chemical Engineering. Working alongside his teachers and peers, Nickolas has developed and is testing several innovative approaches to further the industry of biofuel.

Email: Nchen@oapb.org

Contact number: 1 (561)267-0894

Notes/Comments: