# The influence of thermic treatment on viscoelastic properties of squash (Cucurbita moschata) pulp

Somaris E. Quintana, MSc, Group IFCRA, Colombia

Ronald M Marsiglia, MSc, Group CTS, Colombia

Edilbert E. Torregroza, PhD, Group CTS, Colombia

Jose M. Franco, PhD, Group Complex Fluid, Spain

Luis A. García-Zapateiro, PhD, Group IFCRA, Colombia

### Abstract (300 word limit)

Statement of the Problem: The physicochemical properties of vegetables and fruits can change due to the storage condition, the operations and the thermic treatment applied. So in the food industry, i.e. 20ºC are used in storage and 60ºC in processes such as pasteurization and others. During pumping, in-pipe flow, mixing and stirring of liquid-like foods, shear rates in the range of 10–1000 s-1 may be experienced. Therefore, rheological experiments should preferably be done within the frequency and stress range that encompasses most applications. The purpose of this study is to describe the effect of thermic treatment on the viscoelastic properties of squash (Cucurbita moschata) pulp. Methodology & Theoretical Orientation: Rheological measurements were carried out by using a stress-controlled rheometer Haake Mars 60 (TA Instruments, England), using roughened plate-plate geometries (diameter 35mm, gap 1 mm, roughness 0,5mm). Prior to measurement, all samples were allowed to stand for 600 seconds to relaxation of the same, the temperature of the samples was assured by Peltier system installed in the inferior plate of the rheometer. Oscillatory stress sweeps between 0.001 to 1000 Pa were performed to determine the linear viscoelastic range at a frequency of 1Hz. Then, a frequency sweep between 0.01 and 100 Hz was performed to determine the mechanical spectra. All the rheological tests were performed at temperatures of 20, 40, 60, 80 and 100ºC. Findings: The behavior of storage modulus G’ (elastic behavior) is higher than and loss modulus G’’ (viscous modulus) in the frequency range of temperatures applied. According to this, the viscous and elastic properties of squash pulp are significantly different for temperatures of 20, 40, 60, 80 and 100ºC. On the other hand, no significant differences were found for 40 and 60ºC. So the lineal viscoelastic behavior of squash pulps is similar to mango, cherry and borojó; And is influenced by the temperature Conclusion & Significance: The present work has evaluated the viscoelastic properties of squash pulp. Product has shown a weak behavior, with storage modulus higher than loss modulus in the evaluated frequency.

### Image

### Figure 1: Effects of thermic temperatures on pulps of squash (Cucurbita moschata) on storage and loss modulus

Recent Publications (minimum 5)

- Tonon R.V., Alexandre D., Hubinger M.D., Cunha R.L. (2009). Steady and dynamic shear rheological properties of açai pulp (Euterpe oleraceae Mart.) Journal of Food Engineering. 92, 4: 425–431

- Solange Vandresen, Mara G.N. Quadri, José A.R. de Souza, Dachamir Hotza, Temperature effect on the rheological behavior of carrot juices. Journal of Food Engineering 92 (2009) 269–274

- Pedro E. D. Augusto a,b*; Víctor Falguera c; Marcelo Cristianini a; Albert Ibarz. (2011). Viscoelastic properties of tomato juice. Procedia Food Science 1: 589 – 593

- Diaz-Ocampo, R., L.A. Garcia-Zapateiro, J.M. Franco-Gomez y C. Vallejo-Torres. (2012). Caracterización bromatológica, fisicoquímica microbiológica y reológica de la pulpa de borojó (Borojoa patinoi Cuatrec). Ciencia y Tecnologia 5(1), 17-24

- Pelegrine, D.H., F. Silva y C. Gasperrato.(2002) Rheological behavior of mango and pineapple pulps. Lebensmittel-Wissenschaft und-Technolo-gie 35(1):645-648.

Biography (150 word limit)

Luis A. García-Zapateiro doctor in chemical engineering, has his expertise in food product design, complex fluid and rheology. The Principal investigations are in vegetable oils biodegradable, estolides and greases. In the food product has performed emulsions, hydrocolloids from fruit, isolated protein from fish and vegetable seeds such as beans. He has worked in the department the complex fluid of Huelva University (Spain) and currently he works as Postgraduate Department Head Faculty of Engineering of the University of Cartagena (Colombia), and also he is director of the research group IFCRA. Email: lgarciaz@unicartagena.edu.co

Notes/Comments: This investigation is development by sources of FONDO NACIONAL DE FINANCIAMIENTO PARA LA CIENCIA, LA TECNOLOGÍA Y LA INNOVACIÓN FRANCISCO JOSÉ DE CALDAS – Colciencias. Project  0487-2014 code 1107-667-44997