# Modelling and experimental characterization of the damping behavior of a novel composite Nitinol-Aluminum sheet metal topology

Nursultan Alzhanov, MSc, School of Engineering and Digital Sciences, Nazarbayev University, Kazakhstan

### Abstract

### To increase damping capability as have shape memory alloys (SMA) and keeps the high stiffness as in aluminum, novel composite material is proposed. With passive damping to isolate vibrations by using superelastic property of SMA and fusing aluminum metal matrix on SMA by special technique to achieve high stiffness and at the same time high damping capacity. A technique which is suitable to produce composite material is hot-pressing for synthesizing aluminum metal matrix with reinforced directionally aligned nickel-titanium composite. In this paper, effect of a damping capacity on a whole structure by adding a damping material in structural component was investigated. The damping capacity of novel composite material was determined through two numerical simulation cases and one experiment of free vibration for newly produced composite material. Simulations on bending and free vibration were accomplished by changing %wt ratio of nitinol in overall composition and calculating its loss of energy on each cycle. The damping capacity of an experiment for composite material was calculated by conventional logarithmic decrement. The results of loss factor for simulation and testing experiment were compared. Additionally, SEM (scanning electron microscopy) analysis is presented to show faying surfaces of composite material after hot pressing effect and surface interaction between aluminum and nitinol.

Key words: shape-memory alloys, superelastic effect, experimental tests

Biography

Nursultan Alzhanov has her expertise in evaluation and passion in improving the spacecraft design. His open and contextual evaluation model based on superelastic property of shape memory alloys allow to passively dampen low vibrations impact on a spacecraft structure and on its payload.

Presenting author details

Full name (As per the passport): Nursultan Alzhanov

University/Industry/Organization name: Nazarbayev University

Designation (Director/Prof/Assistant or Associate prof/Researcher/Student): Christos Spitas

Country: Kazakhstan

Mobile number (country code – number): +7 708 173 13 37

Email: nursultan.alzhanov@nu.edu.kz

Track number:  Aerospace and mechanical engineering (I could not find number)
Category (Oral presentation/ Poster presentation/Young researcher forum/E-poster): video presentation

Research interest: dynamics of vibrations, nonlinear materials.

Year of birth: 1993

Note: Please fill up the Red mark fields so that it would be easier for us to cite your details in the conference particulars. (Please do not forget to fix your photo and Logo above the title)