Verification of propranologenesis on catalysed platinum based on Alumina

Abstract

In this study, the kinetics of hydrogenation of propion on a catalyzed platinum based on aluminum have been verified. The experiments were carried out on the propagation of hydrogenation of propylene on a commercial platinum-based alkaline aluminum containing Pt of 0.6%, 0.8% Sn and 0.85% of K, in a 10-millimeter diameter quartz laboratory at three temperatures of 580, 600 and 620 ° C, and an atmospheric pressure of molar ratio Hydrogen was made to Propane 0.8 and for each experiment, the conversion rate was calculated based on laboratory results. According to a previous study carried out in this field, the resistance of external and internal mass transfer is in a test case and can be considered. Several kinetic models were extracted based on hypothetical mechanisms and the coefficients of these models were determined based on laboratory results and optimization performed by MATLAB software and minimizing the sum of squared errors. For evaluation of the cystic model, the F-test was used and based on this method, for each model, an analysis of variance was made. The results of this method indicate that all models are statistically acceptable. Finally, a number of models were selected based on the parameters calculated in the variance analysis table, such as Mean of Squares Erro, as the Hyberter model.

Keywords: Kinetics, Dehydrogenation, Propane, Propylene, Pt / Sn-Al2O3 Catalyst