Finite Element Analysis of Buckling- Controlled Braces

Mustafa Mahamid1, Ataollah Taghipour Anvari1, Jay Shen2, Onur Seker2

1University of Illinois at Chicago, IL 60607, USA

2Iowa State university, IA 50011, USA

The global buckling of conventional braced frames within steel structures is observed during cyclic or earthquake loading. However, recent studies have shown that the premature fracture would occur in the braces due to loss of strength and local buckling which may lead to instabilities in the braced frames. Also, the unsymmetrical and unstable response of conventional braces in tension and compression in special concentrically braced frames leads to the generation of unbalanced loads in the intersected girders and causes additional loads on connections. Various configurations were developed to prevent the buckling of the braces in compression such as buckling restrained braces (BRB) in which continuous lateral support is provided by adding a steel case and mortar around the brace section. In a recently improved configuration, Buckling Controlled Braces (BCB) that utilizes a structural steel Tube-in-Tube (TinT) brace, have shown a stable and symmetrical hysteresis under cyclic loadings with higher energy dissipation capacity. In this type of braces an outer tube section, which does not contribute to load-carrying capacity of the brace, limits the buckling of the inner tube to delay the fracture of the brace under cyclic loadings. The objective of this study is to present a developed finite element model to investigate the various key parameters that affect the response of BCBs. The FE models were compared with experimental results and were validated. Key modeling parameters such as cyclic loading, effect of contact between the two sections, nonlinearity and strength loss of steel material and material models were considered to create an accurate simulation of the Buckling Controlled Braces.

Biography

Dr. Mustafa Mahamid PhD, SE, PE, P.Eng, F.ASCE, F.SEI is a Clinical Associate Professor at the University of Illinois at Chicago and a structural Engineering consultant for multiple structural engineering firms in the US and abroad. He worked as a structural engineer at GRAEF and Skidmore, Owings &amp; Merrill (SOM) in Chicago. He authored and coauthored numerous journal and conference papers, and presents at engineering conferences in the U.S., Europe, and the Middle East.

Mr. Ataollah Taghipour Anvari is a graduate student in the civil and materials engineering at university of Illinois at Chicago. His research interest is design of steel and reinforced structures under various types of loads such as earthquake and fire. Currently, he is studying the effect of partial loss of fireproofing material on fire resistance of the structural elements as well as the behavior of buckling controlled braces under cyclic loads.

Dr. Jay Shen PhD, PE, SE is Structural Engineering Group Coordinator and Associate Professor. His expertise and research interests are in the areas of inelastic behavior, collapse mechanism of steel structures and earthquake engineering, including cyclic behavior and earthquake-resistant design of steel buildings with moment and braced frames, wind and earthquake induced dynamic problems in steel structures, and seismic retrofit of steel building structures. His current research includes model-based simulations of large-scale structures; analytical and experimental studies of collapse and failures in steel buildings with moment and braced frames under natural hazards; and performance-based design of steel structures using energy approach.

Dr. Onur Seker earned his M.Sc. degree from Yildiz Technical University. He studied structural engineering at Iowa State University. His research included various projects related to the seismic behavior of steel structures with a special emphasis on the two major lateral resisting systems: Braced frames and moment-resisting steel frames. He has published eight peer-reviewed journal papers, a technical report and several conference papers. In April 2016, ISU awarded his studies a Research Excellence Award. After completing his Ph.D., he worked in a post-doctoral research associate position at Iowa State University. He is an assistant professor in the Department of Civil Engineering at MEF University.

Presenting author details 
Full name: Mustafa Mahamid	
Contact number: +1 312-355-0364

Linked In account: https://www.linkedin.com/in/mustafa-mahamid-phd-se-pe-f-sei-f-asce-88b5778/

Session name/ number:
Category: Oral presentation