# Delamination damage detection of composite beam using Response Surface Method (RSM)

# Swarup K. Barman, Research Scholar, IIT Kharagpur, India

Dipak Kumar Maiti, Professor, IIT Kharagpur, INDIA

Damodar Maity, Professor, IIT Kharagpur, INDIA

### Abstract (300 word limit)

Composite materials have gained wide importance in various industrial, civil, mechanical, aerospace, naval and military applications. Higher stiffness along with light weight give composite materials an edge over other traditional materials for such applications. However, composites are very sensitive to defects, induced during manufacturing or service period. Delamination is one of the major defects commonly encountered in composite materials. Once induced in the structure, it can propagate very fast leading the structure to a cataclysmic failure in later period. So, this kind of damages need to be detected in an early stage to avoid such occurrences. Throughout the years many researchers have tried to detect delamination damages by solving the vibrational response based inverse problem. Here, the authors have formulated the composite beam structure using 8-noded isoparametric elements based on layer-wise theory. Heaviside step function has been used to model the delaminated part. Combined natural frequencies and modeshapes are used as damage indicator. Instead of using finite model updating based inverse technique, Response Surface Method (RSM) is used. First the frequency and modeshapes based objective function is calculated for varying size and location of delmaintions using the finite element method. The interface of damage is kept constant. Then, MINITAB software is used to fit a surface of the response (objective function) as a function of location and size of the delmination. Lastly, unified particle swarm optimization (UPSO) is used to minimize the fitted surface to locate the location and size of the delamination. Present method is capable of reducing the computational effort significantly by eliminating the use of rigorous finite element model updating in every iteration during the optimization stage. Although at present this method is capable of handling only single delamination, it has greater potential in the field of damage detection in composite structure.

### Image

Recent Publications (minimum 5)

- Chaudhry, Z., and Ganino, A. J. (1994). “Damage Detection Using Neural Networks: An Initial Experimental Study on Debonded Beams.” Journal of Intelligent Material Systems and Structures, 5, 585–589.

- Ratcliffe, C. P., and Bagaria, W. J. (1998). “Vibration technique for locating delamination in a composite beam.” AIAA Journal, 36(6), 1074–1077.

- Yoon, M. K., Heider, D., Gillespie, J. W., Ratcliffe, C. P., and Crane, R. M. (2010). “Local damage detection with the global fitting method using operating deflection shape data.” Journal of Nondestructive Evaluation, 29(1), 25–37.

- Zhang, Z., Shankar, K., Ray, T., Morozov, E. V., and Tahtali, M. (2013). “Vibration-based inverse algorithms for detection of delamination in composites.” Composite Structures, 102, 226–236.

- Torabi, K., Shariati-Nia, M., and Heidari-Rarani, M. (2017). “Moving support technique for delaminatoin detection in laminated composite beams using the first natural frequency.” Journal of Reinforced Plastics and Composites, 36(15), 1116–1128

Biography (150 word limit)

He did B.E. in civil engineering from Bengal Engineering College, Shibpur under Calcutta University. Subsequently he acquired M.Tech and PhD degrees from Department of Aerospace Engineering, IIT Kharagpur. After a short stay at IIT Bombay as Senior Research Engineer he worked at Aeronautical Development Agency, Bangalore as scientist for over six and half years. He joined department of Aerospace Engineering, IIT Kharagpur as a faculty member since October 2004. He has published over 70 international journal papers, over 60 national and international conference papers handled several research projects and guided several PhD students. His primary research areas are analysis of composite structures employing various higher-order shear deformation theories, smart structures, aeroelasticity, structural health monitoring, etc.

Email: dkmaiti@aero.iitkgp.ernet.in