A study on the transient seepage analysis during diurnal tidal cycle

Smita Tung, Ph. D, Guest Lecturer, Jadavpur University

### Abstract

The Sundarban delta was formed at the confluence of the rivers Ganga, Bramhaputra and their distributaries feeding into the Bay of Bengal. The total length of existing earthen embankments in Indian Sundarban is approximately 3520 km. The transient seepage may play a predominant role in the stability of such structures in areas, which are exposed to high variation of cyclic tidal heads on diurnal basis. A single tidal cycle consists of rising and falling of water level during Rise-Up and Draw-Down half cycle time, respectively. A numerical and experimental model has been developed based on the field problem.

The present study is formulated to analyse the dynamics of the phreatic surface of an earthen embankment during transient stage including rise-up and draw-down condition of a single and multiple tidal cycle by generation of a physical and numerical model based on the study area. An experimental study in Centrifuge and Finite element analysis using GEOSTUDIO software has been undertaken to investigate the effect of tidal cycles on earthen embankments. It has been seen that as the embankment soil exposed to continuous multiple tidal cycles consisting of Rise-Up and Draw-down, the embankment soil encounters cyclic settlement and rebound. The effect of the higher number of cycles results serviceability failure by creating the higher chances of permanent deformation. It has been observed that transient seepage induced by Cyclic Tidal Head has a dominant effect on the pore water pressure distribution within an earthen embankment. In our present investigation it has been studied that in a typical Sundarban earthen embankment, the Lower-Slope is more prone to serviceability failure compared to the Berm and Upper-Slope. This is due to tide induced cyclic hydrostatic loading on the riverside part of the embankment resulting in cyclic settlement and rebound of the plastic soil, leading to permanent deformation.

### Image

Recent Publications

- S Tung, G Bhandari, S P  Mukherjee (2016) “Behavior of sheet pile as seepage cutoff below earthen dam”, International Journal of Geotechnical Engineering , Taylor and Francis Group, Vol. 10, issue 2, pp. 162-173

- S Tung, G Bhandari, S P  Mukherjee, A rotaru (2016) “Stability analysis of the earth embankments subjected to natural cyclic processes”, advanced engineering forum , issn: 2234- 991x, vol. 21, pp 389-396 , trans tech Publications, switzerland.

- S Tung, G Bhandari, S P  Mukherjee (2016) “Effect of seepage cutoff on some important aspects of earthen dam under rise up condition by numerical modelling”, Geotechnics for infrastructure development -2016, Kolkata

- S Tung, G Bhandari, S P  Mukherjee (2016)  “stability of earthen embankment with clay core under tidal fluctuation”, IGC-2016.

- J Dey, S Tung, G Bhandari (2016) “Temporal variation of pore water pressure distribution and saturated zone in a tidal water front embankment”Accepted for the publication in International Journal of Hydraulic Engineering (Vol.5, No.1).

Biography

Smita Tung received the B. Tech degree in Civil Engineering and M.E. degree in Geotechnical Specialization. Her Ph. D research area focused on effect of tide induced transient seepage on existing sundarban earthen embankments. Her innovative research work exploring new techniques by improving stability of leeve that leads to a great positive impact on socio economical livelihood in creek side habitant in sundarban. In 2015 she joined as a Guest Lecturer in the Department of Civil Engineering, Jadavpur University. Her area of research interest fascinate on River Mechanism, coastal Engineering, Cloud motion prediction and soil structure interaction.

Email: smita1989smita@gmail.com

Notes/Comments: