# Overall Index of Pollution (OIP) based Water Quality Index- An instrument for water Resources management

Anoop Kumar Shukla, Research Scholar, IIT Roorkee, India    C.S.P. Ojha, Professor, IIT Roorkee, India                                    R.D. Garg, Associate Professor, IIT Roorkee, India

### Abstract

Water quality is an important indicator of river basin health. For better river basin planning and management, the information is required on the state of water quality and the changes occurring into it. GIS are efficient aids for such study. Water quality indices (WQI) were introduced with the aim of reducing great amount of parameters into a simpler expression and enabling easy interpretation of monitoring data. In this study, an attempt has been made to devise a methodology to integrate the WQI with geographic information system (GIS) for an effective interpretation of the quality status of the Ganga River basin. River Ganga basin has been taken as a case study and the physical, chemical and biological analysis has been interpreted using WQI. Final elucidation of the water quality has been done on a map using GIS environment. Water quality parameters such as pH, Dissolved Oxygen (DO), Biological Oxygen Demand (BOD), Turbidity, Fluoride (F), Hardness CaCO3, and Fecal Coliform Bacteria, at different sampling stations across the Ganga river basin were analyzed to monitor the water quality changes in the basin. Based on the results of the analyses, spatial distribution maps of selected water quality parameters were prepared using ArcGIS software. OIP estimates across the river basin demonstrate that the water quality of Uttarkashi and Rishikesh remain in acceptable class for all the three seasons except for July and November 2006. At Rishikesh, the OIP is estimated to be 2.49 in monsoon month and 2.64 in post-monsoon month. These observation stations are surrounded by hills and due to less population, they are not much influenced by human intervention. Allahabad is the most polluted station followed by Kanpur and Varanasi. But the values are much higher at Kanpur and Varanasi. The most inferior water quality with OIP value of 4.18 (polluted class) is observed in the pre-monsoon season of 2010 at Varanasi station. The overall index of pollution (OIP) based on the individual index values were estimated giving the values in terms of pollution indices.

### Methodology

OIP is estimated as the average of all the pollution indices (Pi) for individual water quality parameter considered in this study and is given by the mathematical expression:

OIP= i=1nPin

Where Pi = pollution index for ith parameter. i = 1, 2 … n and n = number of parameters.

References

1. 	Abbasi, S.A. (2002) “Water Quality Indices: State of the Art    Report”, National Institute of Hydrology, Scientific contribution, INCOH/SAR-25/2002.

- Kindu, M., Schneider, T., Teketay, D., and Knoke, T. (2015). “Drivers of land use/land cover changes in Munessa-Shashemene landscape of the south-central highlands of Ethiopia. Environmental Monitoring and Assessment, 187(7): 1-17.

- Li, J., Meng, X., Zhang, Y., Li, J., Xia, L., & Zheng, H. (2015). “Analysis of the temporal and spatial distribution of water quality in China’s major river basins, and trends between 2005 and 2010”. Frontiers of Earth Science, 9(3):463-472.

- Rokbani, M.K., Gueddari, M. and Bouhlila, R. (2011) “Use of geographical Information system and water quality index to assess groundwater quality in El-Khairat deep aquifer (Enfidha, Tunisian Sahel)’’, Iranica journal of Energy and Environment, 2(2):133-144.

- Sargaonker, A. and Deshpande, V. (2003) “Development of an overall index of pollution for surface water based on a general classification scheme in Indian context”, Environmental Monitoring and Assessment, 89:43-67.

Biography

Anoop Kumar Shukla was born in Chitrakoot, Uttar Pradesh, India. He received M. Tech. in Geomatics from JNTU, Hyderabad, India in 2008. From 2012, he is pursuing the Ph.D. degree in Civil Engineering from the Indian Institute of Technology, Roorkee, Uttarakhand, India. His research interest is remote sensing, GIS, water resources management and climate change. He has published 13 papers in international/national refereed journals, 2 in international/ national book chapters and 1 in international conferences. This author became a Student Member of IEEE since 2016. He also received Department of Science and Technology (DST), Young Scientist award in 2014.

Email: anoopgeomatics@gmail.com