- Reliability analysis of roof rainwater harvesting systems in a semi-arid region of sub-Saharan Africa: case study of Mekelle, Ethiopia

- Dilemma of roof rainwater quality: applications of physical and organic treatment methods in a water scarce region of Mekelle, Ethiopia

### ABSTRACT

### Curbing water scarcity problems in semi-arid regions is a top priority for economic and social development. Alternative strategies for sustainable domestic water supply are therefore required to augment water supply with affordable cost and technology. Scant research findings in sub-Saharan Africa have, however, reported inefficiency in adapting to water scarcity problems in the region. Therefore, an investigation of domestic rainwater harvesting (RWH) reliability is important to identify factors affecting the system. We found that rainwater harvesting with the existing traditional RWH systems in Mekelle city was unreliable. Unreliability was attributed to inefficient design (the systems were installed without considering deterministically the stochastic nature of rainfall), family size, water demand, and rooftop area and storage tank size.

- ABSTRACT

The public health field has built a body of literature showing health benefits from improvements in water quality. However, the connection between roof harvested rainwater and health is not well documented especially in developing countries. Understanding the application of locally available treatment methods provides insight into this problem. This paper reports on experimental investigations where rainwater collected from a typical domestic roof in Mekelle, Ethiopia was treated using Moringa stenopetala seed, sand filter and boiling. The quality of the raw and treated roof harvested rainwater was compared against the Ethiopian and World Health Organization pollutants analyzed were total coliforms and turbidity. A significant improvement of turbidity and total coliforms was observed. This implies that application of plant coagulant followed by filtration can sufficiently treat rainwater and can be used as a low-cost

treatment option in water scarce areas. Drinking water standards to investigate its suitability as a supplement for potable water supply.

The Recent Publications (minimum 5)

1. Healthcare Access and Quality Index based on mortality from causes amenable to personal health care in 195 countries and territories, 1990-2015: a novel analysis from the Global Burden of Disease Study 2015. Lancet 2017, 18(17):30818-30818.

2.  Taffere, G. R., A. Beyene, S. A. H. Vuai, J. Gasana and Y. Seleshi (2016). "Reliability analysis of roof rainwater harvesting systems in a semi-arid region of sub-Saharan Africa: case study of Mekelle, Ethiopia." Hydrological Sciences Journal 61(6): 1135-1140.

3. Taffere, G. R., A. Beyene, S. A. H. Vuai, J. Gasana and Y. Seleshi (2016). "Characterization of Atmospheric Bulk Deposition: Implications on the Quality of Rainwater Harvesting Systems in the Semi-Arid City of Mekelle, Northern Ethiopia." Environmental Processes 3(1): 247-261.

4. G.R. Taffere, A. Beyene, S.A.H. Vuai, J. Gasana, Y. Seleshi. Location and time-specific investigation of roof rainwater quality is important to safeguard public health. Desalin Water Treat. (2015, in press), doi:10.1080/19443994.2015. 1107757.

5.  G.R. Taffere, A. Beyene, S. Vuai, J. Gasana and Y. Seleshi "Dilemma of roof rainwater quality: applications of physical and organic treatment methods in a water scarce region of Mekelle, Ethiopia." Urban Water Journal.(2016,inpress),doi:10.1080/1573062x.2016.1176225

Full Name: Getachew Redae Taffere

Program:  PhD in Water and Health: Water and Public Health

Tel Phone: +251-916317476

• In 2001 I received a Diploma in Environmental Health from Debub University, Ethiopia

• In 2005 I received a Bachelor degree in the field of Environmental Health from Jimma University, Ethiopia

• In 2008 I received Master’s degree in the field of Environmental Science from Addis Ababa University, Ethiopia

• In 2012 I received my PhD in Water and Public Health from Addis Ababa University

Currently working as head department of Environmental Health and Behavioral Science at Mekelle University, Ethiopia