Increasing the Efficiency of Wind Turbines Electric Generation Capactiy with an Airfoil Spinner

Amelia Peck

Stonweall Academy, Tolland Connecticut, USA

Abstract

Given the current rate of climate change, renewable energy is vital to the future of our planet. Wind energy is one of the most promising sources of renewables, but currently falls short in efficiency of electrical output when compared to the Betz limit. I have focused on designing different shapes of hubs, or airfoil spinners, to maximize electrical output for small scale wind turbines, through directing the wind more smoothly across the turbine’s nacelle, so the blades have more available wind, and thus more possible kinetic energy. To obtain the most optimal designs, an online modelling software was used to analyze the effect of different shaped spinners on output and wind flow. These were then analyzed based on the Tollmien-Schlichting waves and minimizing the drag coefficient. The Reynolds number was used to calculate the type of wind flow. The best performing spinner shapes were picked for testing, and mounted on a 1-meter, 5 bladed turbine. Based on this, the voltage, amperage and wattage were calculated. The first spinner design improved the average output by 6%; the second improved on average by 11%, when compared to the average outputs of the control group with the standard nacelle. Based on this, the tip speed ratio (TSR), rotations per minute (RPMs), power in the wind, and coefficient of performance (Cp) were calculated to analyze all aspects of their performance. These findings illustrate the significant effect spinners will have on efficiency for small scale turbines, a rapidly growing wind energy sector.

Biography

Amelia Peck is an 18-year old student who has been doing reaserch in the field of wind energy for the last three years. She was a recepient of the Petit Foundation’s Women in Science and Engineering Award, the United Technologies Cooperation’s Award for Excellence in Engineering, and the Arthur Mensing Award for Excellence in the Physical Sciences. In 2016, she represented the United States at the International World Energy, Enviroment, and Engineering Project Olympiad (I-SWEEEP), and recieved a silver medal in the energy category.

Presenting author details 
Full name: Amelia Peck          
Contact number: 8608050214
Twitter account: N/A
Linked In account: linkedin.com/in/ameliapeck

Session name/ number: Renewable and Sustainable Energy 
Category: (Oral presentation/ Poster presentation) Oral Presentation