A Distributed Event-Triggered Control Strategy for DC Microgrids over Industrial Wireless Sensor Networks

Kamyar Mehran, Seyed Amir Alavi, Yang Hao, Ardavan Rahimian

Queen Mary University of London, UK

Design, analysis, and performance evaluation of a distributed event-triggered control for DC microgrids over an industrial wirless sensor network (WSN) is still a challenging problem. The primary objective of this work is to efficiently stabilize the microgrid voltage, and to further balance the energy level of the energy storage (ES) systems communicating via a WSN. The locally-installed distributed controllers are utilized to reduce the number of transmitted packets and battery usage of the installed sensors, based on a proposed event-triggered communication scheme. Also, to reduce the network traffic, an optimal observer is employed which utilizes a modified Kalman consensus filter (KCF) to estimate the state of the DC microgrid via the distributed sensors. Furthermore, in order to effectively provide an intelligent data exchange mechanism for the proposed event-triggered controller, the publish-subscribe communication model is employed to setup a distributed control infrastructure in industrial wireless sensor networks (WSNs). The performance of the proposed control and estimation strategy is validated via the simulations of a DC microgrid composed of renewable energy sources (RESs). The results confirm the appropriateness of the implemented strategy for the optimal utilization of the advanced industrial network architectures in the smart grids.

Biography

Dr Kamyar Mehran is active in the field of energy management/efficiency using wireless sensor networks, intelligent control, fuzzy systems and multi-sensory fusion algorithms with over 25 publications and book chapters in the field. Recent research projects include low-cost conditional monitoring system for aged Lithium-Ion batteries in rural micro-grids (EPSRC GCRF project), and novel intelligent control systems for extending the life-cycle and safety of Li-ion battery packs (Newton fund). He has a successful experience of commercializing an opto-electronic technology using GaN devices from the European FP7 project (with 7 university and commercial partners) and acted as CTO.

Presenting author details 
Full name: Kamyar Mehran
Contact number: 0207 882 6123

Email id: k.mehran@qmul.ac.uk
Category: Oral presentation

Session name/ number: N/A

Research interest: Wireless Sensor Network, Distributed Communication, Distributed Control and State Estimation, Smart Grid, Energy Efficiency.