# Development and Characterization of Bioactive Film Incorporated with Starch Nanocrystals and Different Essential Oils

Neethu Hari & Dr  A. Jayakumaran Nair,

Department of Biotechnology, University of Kerala, Kariavattom campus, Thiruvananthapuram, Kerala, India

### Abstract (300 word limit)

Organic films and coatings, especially those of natural polymers attained more attention in food packaging due to its great versatility in the chemical groups that can be incorporated at surfaces of food items such as meat, fish, Fruits and vegetables (Meng et al., 2009). Among the known biopolymer, chitosan seems to be promising material for the development of bioactive films. The cationic nature of chitosan enhances the film forming capacity of this polymer (Martin et al., 2012). However, films made only from chitosan lack water resistance and have reduced mechanical properties. The functional properties of chitosan films can be improved when chitosan films are combined with nanomaterials. Nanomaterials offer wide applications in the food packaging industry due to the range of advanced functional properties. In the present study, chitosan-based film modified by the addition of starch nanocrystals and Essential oils were used to examine their physical and biological properties. Structural characterization of the modified film was carried out using XRD and SEM analysis. The incorporation of the starch nanocrystals into chitosan-based films enhances the crystallinity of the film. FTIR analysis confirmed the results obtained in this study. The opacity of the film showed a positive correlation (O=6.0) with increased incorporation of orange oil and starch nanocrystals, while moisture content, solubility and swelling nature of the modified film showed a negative correlation. An increased antimicrobial and antioxidant activity was observed for modified chitosan films. Taken together, these results show that the chitosan film modified with starch nanocrystals and essential oil may act as an active packaging film, which could be used to extend the shelf-life of packaged food.

Recent Publications (minimum 5)

- Joseph MM, Nair JB, Adukkadan RN, Hari N, Pillai RK, Nair AJ, Maiti KK1,2, Therakathinal T S (2017)  Exploration of Biogenic Nano-chemobiotics Fabricated by Silver Nanoparticle and Galactoxyloglucan with an Efficient Biodistribution in Solid Tumor Investigated by SERS Fingerprinting. .ACS Appl Mater Interfaces.  14;9(23):19578-19590.

- Koshy L, Anju AL, Harikrishnan S. Kutty VR, Jissa VT, Kurikesu I, Jayachandran P2, Jayakumaran Nair A, Gangaprasad A, Nair GM, Sudhakaran PR (2016) Evaluating genomic DNA extraction methods from human whole blood using endpointand real-time PCR assays. MolBiol Rep. 2017 Feb;44(1):97-108.

- S. Sinimol, A. R. Sarika, and A. Jayakumaran Nair (2016) Diversity and antagonistic potential of marine microbes collected from south-west coast of India. 3 Biotech. 6(1): 7

- Prasad, Roopa& Nair, Ananthakrishnan J & Abraham, Theruvath.(2017) Purification and Characterization of Alkaline Metalloprotease from Bacillus sp. MTCC 9558 and its Application in Bioactive Peptide Production. Journal of Pure and Applied Microbiology. 11. 187-196. 10.22207/JPAM.11.1.24.

- Vijayakumari Nadaraja A, Jayakumaran Nair A, Prameela M, Hari N, Balakrishnan N. Evaluation of currently employed food preservation conditions to tackle biofilm forming food pathogens. J Food Saf. 2017;e12407.

Dr. A. Jayakumaran Nair is working as faculty and Head of the Department of Biotechnology, University of Kerala, Thiruvananthapuram, Kerala, India. He also is ther Director of the centre for genomics and gene Technology, another Institute in the University of Kerala. He is working in the area of Microbial biotechnology, protein expression studies and also in the area of nanobiology and food biotechnology. He also work in the area of Genomics of infectious diseases and metabolic disorders and studying the role of micro RNAs in the disease.  Screening of natural resources for the cytotoxic and anticancerous compounds is his another area of investigations.

Email: jekksnair@gmail.com

Notes/Comments: