Feasible technical and economic solutions for EPA production by Pythium irregulare using vinasse, wastewater, as main raw material

Bruna S Fernandes1, Bruno C. Klein2, Marcelo Zaiat3, José GC Pradella4, Rubens Maciel Filho1

1-School of Chemical Engineering, University of Campinas, Brazil, 2-Bioethanol Science and Technology Laboratory (CTBE), Brazilian Centre of Research in Energy and Materials (CNPEM), Brazil. 3 -Biological Processes Laboratory, Center for Research, Development and Innovation in Environmental Engineering, São Carlos School of Engineering (EESC), University of São Paulo (USP), Brazil, 4-PRBiotec Ltd, Brazil

### Abstract

### Vinasse is a rich carbon source and low-cost feedstock produced in huge amounts from the process of ethanol production. In 2019, the Brazilian Ministry of Agriculture, Livestock, and Food Supply estimates growth of ethanol domestic consumption of 58.8 billion liters, more than double the amount recorded in 2008. This represents the annual production of more than 588 billion liters of vinasse, which is currently used as fertilizer in the sugarcane crop, due to its high concentration of minerals, mainly potassium. However, studies indicate some disadvantages such as the generation of Greenhouse Gas emission during vinasse distribution in the crop, as well as the possibility of contaminating the groundwater and soil. Therefore, the development of programs for sustainable use of vinasse is a priority.

### One profitable process is the bioconversion of vinasse into biocompounds such as lipids, by microorganisms.

### Promising high value-added lipids, for instance polyunsaturated fatty acids (PUFAS), with a predicted market of millions of US$, can be produced using vinasse as carbon source, to guide an innovative e feasible concept for sustainable production.

### In this context, Omega 3 Eicosapentaenoic acid (EPA), a PUFA, not synthetized by humans but an important dietary supplement with a promising market, was produced by Pythium irregulare, an oleaginous Oomycete, able to accumulate large amounts of lipids, and the production was evaluated according to several technical and economic solutions and scenarios.

### The results of this study showed a great alternative for EPA production applied to human consumption, considering different microbial bioreactors configuration, alternative extraction and refinement processes.

### Recent and relevant Publications

- Doelsch, E., Masion, A., Cazevieille, P., Condom, N. (2009). Spectroscopic characterization of organic matter of a soil and vinasse mixture during aerobic or anaerobic incubation. Waste Manag. 29, 1929–1935.

- Grand View Research. (2014). Omega 3 Market Analysis And Segment Forecasts To 2020 [WWW Document]. URL http://www.grandviewresearch.com/industry-analysis/omega-3-market (accessed 5.17.15).

- Liang, Y., Zhao, X., Strait, M., Wen, Z. (2012). Use of dry-milling derived thin stillage for producing eicosapentaenoic acid (EPA) by the fungus Pythium irregulare. Bioresour. Technol. 111, 404–9.

- MAPA - Ministério da Agricultura, P. e A. (2012). Ministério da Agricultura, Pecuária e Abastecimento [WWW Document]. Ministério da Agric. Pecuária e Abast. URL http://www.agricultura.gov.br/ (accessed 5.11.12).

- Oliveira, B. (2015). Vinhaça de cana-de-açucar: fluxo de gases de estufa e comunidades archaea presente no sedimento do canal de distribuição. Escola Superior de Agricultura “Luiz de Queiroz”, Universidade de São Paulo.

- Leman, J., Olsztyn, M. (2009). Lipids, Production. Lipids 393–406.

Biography

Bruna Soares Fernandes graduated from University Federal of São Carlos (Brazil) on chemical engineering. She did master and doctor degree at University of São Paulo (Brazil) on Hydraulic and Sanitation engineering. She worked at Dedini/Paques BV as new technologies leader focused on wastewater treatment. During the four last years, Dr. Fernandes got a position as technical coordinator of R&D project of Ergostech, Sapporo Brewery and Petrobras. Dr. Fernandes and her team developed a process, in which it was patented. Since 2015, she is in a postdoctoral position at Brazilian Centre of Research in Energy and Materials and University of Campinas. The main interests of Dr. Fernandes are waste treatment and bio-products obtainment from biological processes.

Email: brunasofer@hotmail.com