A comprehensive study on cast NiTi and Ti64 alloys for biomedical applications

Khaled M. Ibrahim

Central Metallurgical Research and Development Institute, Egypt

Abstract

A comprehensive study on two biomaterials of NiTi and Ti-6Al-4V (Ti64) was done. Those materials were cast using vacuum arc remelting technique. As-cast structure of Ni-Ti alloy consists of NiTi matrix and some fine precipitates of Ni4Ti3. Ti-6Al-4V alloy showed a structure composed of equiaxed β grains and varied α-phase morphologies. Maximum ultimate compressive strength and reduction in height of 2042 MPa of 18%, respectively, were reported for the cast Ti64 alloy. However, minimum ultimate compressive strength of 1804 MPa and low reduction in height of 3% was obtained for the cast NiTi alloy. Wear rate of both Ni-Ti and Ti-6Al-4V alloys significantly increased at saline solution (0.9% NaCl) condition as compared to dry testing condition. Saline solution harmed the wear resistance of about 2 to 4 times compared to the dry condition. Corrosion rate of NiTi alloy at saline solution (0.9% NaCl) was (0.00038 mm/yr) was almost three times the value of Ti64 alloy (0.000171 mm/yr). The corrosion rate of Ti64 in SBF (0.00024 mm/yr) was lower than Ni-Ti (0.0003 mm/yr).

Biography

Khaled M. Ibrahim is affiliated to the Central Metallurgical R&D Institute, Helwan, Cairo-Egypt.