Assessment of contamination levels of potentially toxic elements in the sediments of an urban industrial and fish landing site in Uganda

Gabson Baguma

University of Nevada, Las Vegas

Abstract

Sediments of an urban Industrial and fish landing site on Lake Victoria in Port Bell, Uganda was investigated to establish the Heavy Metals (HMs) pollution levels and potential health risks to humans and the ecosystem. Sediment samples were collected in triplicate (n=9) from Port bell, digested and analyzed using Atomic Absorption Spectroscopy (AAS). Geochemical indices; Contamination Factor (CF), Geochemical Accumulation Index (Igeo), Pollution Load Index (PLI) and Potential Ecological Risk (PERI) were estimated for the detected hazardous metals. As per the US EPA sediment quality guidelines, the average dose through dermal contact and Hazard Quotient (HQ) was determined to assess the health associated risks with the dredging works at the landing site. The results showed that Cu (6.111 to 7.111 mg/kg) and Pb (40.222 to 44.212 mg/kg) occurred at the highest concentrations. Pollution load indices showed that the contamination levels are not above the threshold as per the US EPA sediment quality guidelines. However, PERI showed considerable pollution of Port bell sediments, particularly with Cd since it had reached severe enrichment in the sediments (CF=11.510). The high contamination level is due to their introduction from anthropogenic sources. The overall results of the analyses revealed that the sediments within the vicinity of the Port Bell Land site were polluted with HMs, Cu, Pb, Cd and Cr. This is due to activities within these areas that generated a lot of wastes, ranging from plastics, the use of agrochemicals, preservatives, personal care products, scrap metals, used solders and electrodes that contaminate the landing site with the HMs. The enrichment and contamination factors, PLI and Igeo values showed that the sediments in the study area were polluted with these heavy metals. A comprehensive study of the pollution indices shows that steps should be taken to minimize the impact of these elements.

Biography

Gabson is a Graduate Research Assistant and a student in the Department of Civil & Environmental Engineering and Construction at the University of Nevada Las Vegas and a prospective doctoral student of the University of Nevada Las Vegas. He was previously with the Department of Physical Sciences, School of Natural and Applied Science, Kampala International University and affiliated with NAPRECA-Uganda Chapter. He has conducted research on heavy metal toxicology and natural products. His interests-not limited to food toxicology, analytical, natural products, wastewater treatment and Environmental Chemistry.