Poly(ionic liquid)s for photocatalytic CO2 conversion

Chengcheng Liu

Shandong University, China

Abstract

Poly-Ionic Liquids (PILs) refer to polymers polymerized by Ionic Liquids (ILs) monomers. ILs and PILs have been widely used in CO2 conversion. Their ionic microenvironment and special hydrogen bonds can enrich and activate CO2 efficiently, thus facilitating the conversion of CO2 under mild conditions. At present, the applications of PILs in CO2 conversion mainly focus on thermal catalysis and electrocatalytic CO2 conversion. Their applications in photocatalytic CO2 conversion are rarely reported. Photocatalytic CO2 conversion can convert CO2 into high value-added fuel and store solar energy in the fuel, which is one of the important means to overcome the energy crisis and reduce environmental pollution. However, due to the high C=O dissociation energy of CO2, photocatalytic CO2 conversion still faces the problems of low conversion rate and serious hydrogen evolution side reactions. Due to its unique ion pair structure and strong structural design ability, PILs have great potentials in photocatalytic CO2 conversion. The main limiting factor for applications in photocatalysis is the poor photoexcitation ability of PILs. In the previous studies, the author found that the introduction of electron donor-acceptor structure into PILs can significantly improve the separation and migration rate of photogenerated carriers in the PILs structure [Figure 1]. The synthesized PILs show high catalytic activity in both photocatalytic CO2 reduction and photo-driven CO2 cycloaddition reactions. Through the transient absorption and other spectral studies combined with the Density Functional Theory (DFT) and Time-Dependent Density Functional Theory (TD-DFT) calculations, it was found that there was a strong built-in electric field in the designed PILs molecules. The built-in electric field could provide a driving force for the separation and directional transmission of photo-carriers in PILs, thus promoting the photo-catalyzed/photo-induced CO2 conversion.

Figure 1. PILs with donor-acceptor structures for photocatalytic CO2 conversion.

Biography

Chengcheng Liu is an associate researcher at the Institute of Frontier and Interdisciplinary Science, Shandong University. She received her PhD degree in materials science under the supervision of Prof. Buxing Han and Jianling Zhang from Institute of Chemistry, Chinese Academy of Sciences (ICCAS) in 2017. Then she worked as a postdoctoral researcher at the University of Manchester (2017-2019). Her research interest is developing functional porous materials toward their applications in energy fields.