Energy and cost analyses of Bottom Waste Cooking Oil (B-WCO) based solid alcohol biofuel and Supernatant Waste Cooking Oil (S-WCO) based biodiesel

Ramin Nabizadeh Nodehi1*, Abbas Norouzian Baghani1, Sodeh Sadjadi2, Kamyar Yaghmaeian1, Amir Hossein Mahvi1, Masud Yunesian2 and IL Garcia3

1Tehran University of Medical Sciences, Iran

2Nuclear Science and Technology Research Institute, Iran

3University of Córdoba, Spain

Abstract

Waste Cooking Oil (WCO) is one of the most available renewable energy sources to produce Solid Alcohol Bio-Fuel (SABF) and biodiesel. The objective of this work is to carry the energy and cost analyses of biodiesel and SABF generation from Supernatant Waste Cooking Oil (S-WCO) and Bottom Waste Cooking Oil (B-WCO) from catering facilities in Iran, respectively. The total energy input/energy output for SABF and biodiesel, respectively, were 31.73/37.18 MJ/L and 34.269/42.161 MJ/L, with the most contribution of B-WCO/S-WCO (78.78/73.573%), pursued by machinery (19.76/18.297%) and ethanol/methanol (0.684/7.467%). The energy ratio for SABF and biodiesel were 1.172 and 1.23, respectively. The renewable energy/nonrenewable energy inputs forms for SABF and biodiesel included 25.07/6.66 MJ/L (or 79/21%) and 25.28/ 8.986 MJ/L (or 73.78/26.22%) of total energy input, respectively. The total cost and revenue for SABF vs. biodiesel were 1.31 vs. 1.35 $/L and 5.62 vs. 2.481 $/L, respectively. The gross return, net return, beneﬁt to cost ratio and productivity SABF vs. biodiesel generation were 4.324 vs. 1.458 $/L, 4.311 vs. 1.132 $/L, 4.294 vs. 1.839 (dimensionless) and 0.782 vs. 0.845 kg/$, respectively. Hence, concerning energy consumption for industrial production, urbanization and the existence of new energy consumption businesses such as the Bitcoin network, B-WCO and S-WCO can be used as a renewable feedstock for energy production such as SABF and biodiesel.

Keywords: Waste cooking oil, Solid alcohol biofuel, Biodiesel, Cost analyses, Energy.

Biography

Ramin Nabizadeh Nodehi is from Iran. He has done his research on Biodiesel.