Quantifying mangrove extent using a combination of optical and radar images in a wetland complex

Daniel Aja

University of Cape Coast, Ghana

Abstract

The classification of mangrove forests in tropical coastal zones, based only on passive remote sensing methods, is hampered by mangrove complexities, topographical considerations and cloud cover effects, among others. This paper reports on a study that combines optical and radar data to address the challenges of distinguishing mangrove stands in cloud-prone regions. Google Earth Engine geospatial processing platform was used to extract multiple scenes of Landsat surface reflectance Tier 1 and synthetic aperture radar (C-band and L-band). The images were enhanced by creating a feature that removes clouds from the optical data and using speckle filters to remove noise from the radar data. The random forest algorithm proved to be a robust and accurate machine learning approach for mangrove classification and assessment. Classification was evaluated using three scenarios: classification of optical data only, classification of radar data only and combination of optical and radar data. Our result revealed that the scenario that combines optical and radar data performed better. Further analysis showed that about 16.9% and 21% of mangrove and other vegetation/wetland cover were lost between 2009 and 2019. Whereas water body and bare land/built-up areas increased by 7% and 45% respectively. Accuracy was evaluated based on the three scenarios. The overall accuracy of 2019 classification was 98.9% (kappa coefficient=0.979), 84.6% (kappa coefficient=0.718) and 99.1% (kappa coefficient=0.984), for classification of optical data only, classification of radar data only and combination of optical and radar data respectively. Overall, combining both imageries reduced the error of omission or commission of mangroves and increased the user’s/producer’s accuracy up to 99%. This study has revealed the potential to map mangrove correctly, enabling on-site conservation practices in the climate change environment.

Biography

Daniel Aja is a passionate environmental scientist. He obtained a BSc in Environmental Management from Ebonyi State University, Nigeria, an MSc in Environmental Science (energy & climate change option) from Addis Ababa University, Ethiopia and a PhD in Integrated Coastal Zone Management from the university of Cape Coast, Ghana. He is a multi-scholarship award winner. His research interests are Geographic Information System (GIS), remote sensing and environmental modeling.