Spectral optical properties of gray matter in human male brain tissue measured at 400–1100 nm

Jamal H. Ali

Borough of Manhattan Community College, USA

Abstract

Statement of the problem: Gray matter, in the outermost layer of the cerebral cortex, plays a significant role in processing important information such as reasoning, intelligence, emotion, memory and language. There is a strong need to investigate neuron densities and their healthy state in a non-invasive way. All the techniques used to calculate neuron density are invasive ones. Determining the optical properties of gray matter is vital for developing NIR non-invasive diagnostic imaging techniques and therapy.

Methodology & theoretical orientation: The gray matter tissues used were fresh and not treated with chemicals. Using Beer's law and the Mie model, the structural properties, such as the density of neurons, in the gray matter of human brain tissue, were investigated for the first time. Sections of gray matter male brain tissues of 300 μm thickness were obtained for the absorption measurements to evaluate the optical properties.

Findings: The absorption spectra of the gray matter tissues obtained showed clear peaks at 550 and 580 nm due to HBO2 and 970 nm due to water. The attenuation coefficients at 580, 800 and 980 nm are 17.75, 11.16 and 11.24 mm-1, respectively. The measured scattering and transport lengths at 800 nm were 0.09 and 0.90 mm, respectively. The possible NIR optical imaging depth was roughly 3.8 mm, determined by the theoretical penetration limit resulting from ballistic and snake photons. The density of neurons in the examined gray matter tissue sample was estimated, roughly 40,000 neurons/mg using a non-invasive technique [Figure 1].

Figure 1. Illustration of photons' journey in a turbid medium such as tissues.

Conclusion: For the first time, we may be able to non-invasively measure the neurons' density in the cerebral cortex's gray matter. This non-invasive technique is an important step toward studying the state of neurons in a person who has the potential for neurodegenerative diseases, allowing appropriate intervention at an early stage of the disease. The calculated scattering efficiency (2.5), scattering cross-section (282.6 µm2) and scattering coefficient (11.16 mm-1) at 800 nm gave a density of roughly 40,000 neurons/mm3. This is the approximate number of neurons per mg (mm3) in our sample's gray matter of human brain tissue.

Biography

Jamal Ali received his BS in Physics from Yarmouk University in Jordan and an MS from the City College of New York (CCNY). He got his master's degree in Science Education from Queens College. He obtained his PhD in Physics from the City University of New York (CUNY), working at the Institute for Ultrafast Spectroscopy and Lasers (IUSL). He has a broad background in physics, with specific training and expertise in photonics techniques to detect hidden objects beneath highly scattering media. He focuses on non-invasive techniques such as Confocal Scanning Imaging, Spectral Polarization Imaging and Time-Resolved measurements to investigate foreign objects hidden beneath highly scattered media. Lately, he has been directing his research toward Spectral & Optical Brain Imaging techniques. He has 16 publications and four patents in physics, engineering and material science.