ANALYTICAL BIOCHEMISTRY 112, 195—203 ( 1981)

# “Western Blotting": Electrophoretic Transfer of Proteins from Sodium Dodecyl Sulfate— Polyacrylamide Gels to Unmodified Nitrocellulose and Radiographic Detection with

# Antibody and Radioiodinated Protein A

W . NEAL BURNETTE'

Fred Hutchinson Cancer Research Center, 1124 Columbia Street, Seattle, Washington 98104

Received May 20, t980

A simple and efficient procedure was employed for the electrophoretic transfer of proteins from sodium dodecyl sulfate-polyacrylamide gels to sheets of pure, unmodified nitrocellu- lose. Immobilised proteins could then be radiographically visualized in situ by reaction with specific antibody and the subsequent binding of radioiodinated Staphylococcus protein A to the immune complexes. The detection of murine leukemia virus antigens in complex cellular lysates was used to demonstrate the efficacy of this technique.

A powerful tool in molecular genetics has been the blotting technique of Southern (1) in which electrophoretically fractionated DNA can be immobilized onto nitrocellu- lose filters and used to examine comple- mentary sequences by hybridization in situ. An adaptation of the “Southern” blot is the covalent attachment of fractionated RNA (or DNA) to diazobenzyloxymethyl paper (DBM paper)° in order to probe for comple- mentary DNA sequences (“Northern” blotting) (2). Although various proteins have been detected after fractionation on polyacrylamide gels using enzyme sub- strates (3) or specific antibody (4), only re- cently have attempts been made to immobi- lize gel-fractionated proteins on a solid phase (5—7). These techniques involve ei-

' Present address: Tumor Virology Laboratory, The Salk Institute, Post Office Box 85800, San Diego, Calif. 92138.

- Abbreviations used: DBM paper, diazobenzyloxy- methyl paper; MuLV, murine leukemia virus; SDS, sodium dodecyl sulfate; IPA, radioiodinated Staphylo- coccus protein A; NP-40, Nonidet P-40; 2-DGE, two- dimensional gel electrophoresis; BSA, bovine serum albumin; lgG, immunoglobulin G.

195

ther passive diffusion or the preparation and activation of DBM paper to achieve co- valent immobilization.

Our laboratory has been interested in finding a simple yet reproducible transfer technique for use in the immunological de- tection and characterization of the products encoded by murine leukemia virus (MuLV) genomes. In this regard, Towbin et al. (8) recently described an elegant and straight- forward method for the electrophoretic transfer of ribosomal proteins from polyac- rylamide—urea gels to sheets of unmodified nitrocellulose and the radioautographic de- tection of specific antigens on such gel rep- licas with radiolabeled antibodies. Most types of biochemical and immunochemical analyses, however, make use of sodium do- decyl sulfate (SDS)—polyacrylamide gel electrophoresis to identify and characterize proteins by their relative molecular weights. As described below, it was found that the rate of electrophoretic elution of proteins from SDS-containing gels was it- self molecular weight dependent. By mak- ing certain adaptations to the transfer method of Towbin et al. (8), it was possible

0003-2697/8t /060195-09502.00/0

Copyright0 1981 by Academic Press, Inc.

All rights or’ reproduction in any form reserved.

to achieve essentially complete and quan- titative elution of most proteins from SDS—gels to nitrocellulose with no loss in electrophoretic resolution. The use of ra-

dioiodinated StOR!¥! C • Cus pr otein A (IPA), instead of radiolabeled primary or secondary antibodies, greatly simplified and enhanced the radioautographic detection of

immune complexes on blots reacted with specific immune sera.

In addition to the general technique of electrophoretic transfer and immunological detection, various applications of the method are discussed.  Certain  problems are also identified, particularly those re- garding the reuse of the gel blots, replica distortion, and antigen denaturation. With due respect to Southern (1), the established tradition of “geographic” naming of trans- fer  techniques  (“Southern,”   “Northern”) is continued; the method described in this manuscript is referred to as “Western” blotting.

MATERIALS

AKR MuLV-infected C57BL/6 E fi G2 cells and Moloney MuLV-infected C3H cells were donated by R. Nowinski of this Center. "C-Labeled molecular-weight- marker proteins and sodium [' 25 I]iodide were purchased from New England Nu- clear (Boston, Mass.). Staphylococcus pro- tein A from Pharmacia (Piscataway, N. J.) was radioiodinated as described by Hunter (9). Generally, IPA can be stored in ali- quots for up to 2 weeks at — 70°C in solu- tions containing 5% bovine serum albumin without significant loss in reactivity toward immunoglobulin. Acrylamide and bisacryl- amide were from Polysciences (Warring- ton, Pa.) and Sequanal-grade SDS was from Pierce Chemical Company (Rockford, Ill.). Grade 470 filter paper and sheets of nitro- cellulose of various pore sizes were pur- chased from Schleicher and Schuell (S&S, Keene, N. H.). The Canaleo gel destaining apparatus is sold by Miles Laboratories

(Elkhart, Ind.); only the electrophoresis chamber and plate electrodes (sold sepa- rately) are necessary for the transfer tech- nique. Gel electrophoretic apparatuses were manufactured to speeificationi by Biocraft (Peekskill, N. Y.). Rabbit anti-p30 sera were generously provided by H. Fan of the Salk Institute and by J. Ihle of the Frederick Cancer Research Center.

METHODS

Sample preparation. AKR MuLV-in- feeted C57BL/6 E 6 G2 cells and Moloney MuLV-infected C3H cells were washed three times in phosphate-buffered saline. Cell pellets, collected by centrifugation, were suspended at a density of no greater than 20—30 x ltP cells/ml in a lysis buffer

(10) containing 0.5 M urea, 2Wc Nonidet P- 40 (NP-40; Shell Chemical Co., West Or- ange, N. J.), 2'7o Ampholine pH 3.5—10 (LKB Instruments, Stockholm) and 5% 2- mercaptoethanol. The cells were frozen and thawed three times and insoluble debris re- moved by centrifugation for 10 min in an Eppendorf centrifuge. Lysates were stored at —70°C until used. These samples could be applied directly to isoelectric focusing gels for two-dimensional gel electrophoresis (2-DGE). For SDS gel electrophoresis, small sample volumes could be diluted directly into SDS—gel sample buffer ( l l ) or, for larger sample volumes, could be precipitated with cold trichloroacetic acid, the precipitates collected by centrifugation, and washed in cold acetone before resuspension in sample buffer.

Gel electrophoresis. SDS gel electropho- resis was performed in 10, 12.5, or 5-20a linear gradient acrylamide slab gels as pre- viously described ( 11,12). 2-DGE is a cath- ode-directed isotachophoretic step coupled with SDS gel electrophoresis which has been used in this laboratory for the analysis of monoclonal immunoglobulins (13).

Electrophoretic transfer. The transfer procedure has been adapted from Towbin

et al. (8) for the quantitative recovery of proteins from SDS-containing polyacryl- amide gels. Briefly, a “sandwich” is pre- pared with the following successive layers:

(i) a porous polyethylene sheet (Bel-Art),

13.5 x 14 cm and 1.6 mm thick; (ii) three thicknesses of filter paper (S&S grade 470),

13.5 x 10 cm; (iii) the SDS —polyacryl- amide slab ge1 (13.5 x 9.5 cm with the stacking gel removed); (iv) a nitrocellulose sheet (S&S BA83, 0.2 pm) cut to the size of the gel; (v) three more thicknesses of filter paper; and finally (vi) another sheet of po- rous polyethylene. All components in con- tact with the slab gel are prewetted in the electrode solution composed of 20 mM Tris base, 150 mv glycine, and 20Po methanol (8). This solution should be degassed briefly under vacuum before use. The sand- wich is secured with thick rubber bands and inserted between the electrodes of the Can- alco gel destainer with the nitrocellulose toward the anode. The chamber is filled with electrode solution and electrophoretic transfer accomplished at 6—8 V/cm (with respect to electrode separation) for 16—22 h using a Buchler 3-1155 power supply. Re- circulation of the electrode solution is not necessary.

Staining, radioautography, and fluorog- raphy. For direct visualization of unlabeled proteins, the nitrocellulose sheet may be stained for 5 min in a solution containing 0.2Wo Coomassie brilliant blue R-250, 40% methanol, and 10% acetic acid. Rapid de- staining is accomplished in 90% methanol and 2% acetic acid as descñbed (8,14). Care must be exercised during destaining be- cause the nitrocellulose will disintegrate if left in the acidic methanol for much longer than 5 min. The destained sheet is rinsed in water and blotted dry for 1—2 h between sheets of S&S grade 470 paper. Hot air dry- ing should be avoided. Protein preparations radiolabeled with [35S]methionine, ” 5I, or to high specific activities with '"C-labeled precursors may be directly visualized by ra- dioautography after drying the nitrocellu-

lose replica. ' 4C -Labeled and °H-labeled proteins may be visualized by fluorography utilizing the fluorographic cocktail “En- hance” (New England Nuclear), followed by intensifying screen-enhanced radioau- tography (15).

Binding of antibod y and IPA to nitrocel- lulose-immobilised proteins. Immediately following transfer, the nitrocellulose sheet is immersed in a solution containing 0.9P NaCl, 10 mM Tris —HC1, pH 7.4 (Tris—sa- line), and She fraction V bovine serum albu- min (BSA), and incubated at 40°C for 30 min on a rocking platform. The sheet is transferred to a fresh solution of Tris —sa- line with BSA containing the appropriate antiserum and incubated for 90 min at room temperature on a rocking platform. The ni- trocellulose is then washed with rocking for 10 min in 200 ml of Tris—sa)ine without BSA, for 20 min in two changes of 200 ml each of Tris—saline containing 0.05% NP- 40, and again for 10 min in 200 ml of Tris — saline alone. The sheet is immersed in fresh Tris—saline with 5Pr BSA containing 2— 5 x ltP cpm of IPA/ml. Binding of IPA is allowed to occur for 30 min with rocking at room temperature. The radioactive solution is aspirated and the nitrocellulose sheet again rinsed as described above. The sheet is briefly blotted with paper towels, wrapped in Glad-Wrap, and exposed at

— 70°C to Kodak XR film utilizing a GAFMED Rarex Mid Speed intensifying screen.

RESULTS AND DISCUSSION

Efficienc y of Electrophoretic Transfer and Immobilization of Proteins

Figure 1 shows the indirect radioauto- graph of "C-labeled proteins on unmodified nitrocellulose following their transfer at 8 V/cm for 22 h from a 12.5% polyacrylamide

—SDS gel. Compared to the electrophoretic transfer of proteins to DBM paper, Western blotting with unmodified nitrocellulose gives markedly better resolution, is signifi-

9K4 Kg—

30 Kd —

I 2.3 Kd —•

FIG. 1. WBStCfn blot of '•C-labeled proteins. Ap- proximately 6 nCi each of '•C-labeled phosphorylate b, bovine serum albumin, ovalbumin, carbonic anhy- drase, and cytochrome c were fractionated in a 12.5Po polyacrylamide SDS—gel. Following electrophoresis, the proteins were transferred to 0.20 pxi nitrocellulose paper as described in the test and the blot exposed to Kodak SR film.

# cantly easier to perform, and does not re-

amount necessary for detection by even less-sensitive staining methods (e.g. , Coo- massie brilliant blue, amido black).

In preliminary experiments it was ap- parent that an electrophoretic blotting time of 1 h at 6—8 V/cm, as suggested by Towbin et al. (8) and Bittner et al. (7), was not suffi- cient for complete transport of most pro- teins out of SDS-containing polyacrylamide gels. The rate of transfer is, in fact, depen- dent upon the apparent molecular weight of individual proteins, i.e., lower-molecular- weight polypeptides leave the gel and are deposited on the nitrocellulose faster than higher-molecular-weight proteins at a given accelerating voltage. Figure 2 qualitatively illustrates the necessity of longer blotting times. The upper panels of Fig. 2 show the residual radioactivity remaining in 10% polyacrylamide—SDS gels after transfer at 8 V/cm for 0, 1, 4, 12, and 22 h. The lower panels are the corresponding nitrocellulose blots. Quantitative transfer as a function of molecular weight can be demonstrated by cutting the appropriate radiolabeled bands

TIME OF TRANSFER ( h )

# quire preequilibration of the gel nor diazoti- zation of the nitrocellulose (R. Eisenman and W. Mason, personal communication). Six different grades of S&S nitrocellulose paper, with pore sizes ranging from 0.025 to

0.45 pm, were examined for efficiency of protein immobilization. No significant dif- ference was found among them in resolu- tion or in effective adsorption of marker proteins phosphorylase b (97.4 kdalton) bo-

0

OELS

4	12	22

# vine serum albumin (66.3 kdalton), ovalbu- min (43 kdalton), and carbonic anhydrase (30 kdalton). However, the adsorption of cytochrome c (12.5 kdalton) to 0.45 pm ni- trocellose was consistently 20—30% less than to other nitrocelluloses. Conse- quently, nitrocellulose with a pore size of

0.2 pm (S&S BA83) was employed for all subsequent studies. This nitrocellulose paper exhibited a protein adsorption capac- ity of about 2 pm/mm°, far exceeding the

BLOTS	"	”	"

Fic. 2. Transfer of proteins as a function of electro- phoresis time. '•C-Labeled marker proteins were frac- tionatod by l0& SDS gel electrophoresis and electro- phonetically transferred to nitrmllulmc at 8 V/cm for the times indicated. The upper panels are radioauto- grams showing the residual radioactivity remaining in the gels after transfer. The lower panels are the corre- sponding blots for each time point.

from each of the blots and determining ra- dioactivity by liquid scintillation counting. Figure 3 illustrates the kinetics of transfer for two proteins, phosphorylase b (97.4 kdalton) and carbonic anhydrase (30 kdal- ton). It can be seen that quantitative trans- fer of both proteins is only approached after 12 h. Mass dependence of transfer is not affected by presoaking the gel before blot- ting to remove residual detergent. Indeed, equilibration in a solution other than the electrode solution can result in swelling or

qP95/85

P75*

Pr65

A  B

97.4Kd

66.3Kd

43 Kd 3O Kd

shrinking of the gel during blotting with consequent distortion of the replica pattern (see below). For 1.5-mm-thick gels, there- fore, 20—22 h of electrophoresis at 6—8 V/cm is sufficient for greater than 90% transfer of all proteins up to about 100,000 daltons.

Accelerating voltages higher than 10 V/cm are not recommended for electro- phoretic transfer because of the attendant joule heating of the gel. Constant-current electrophoresis is also satisfactory if the voltage limit is not exceeded. Although there is no separation of the cathodic and anodic chambers in the Canalco destainer,

CARBO«i iC

ANH Y OR ASE

PHOSPH0R7bASE h

w 60 -

40

TO

4	£I	I2	16	20

T I ME OF TRANSFER ( h )

Fic. 3. Electrophoretic transfer time as a function of molecular weight. “C-Labeled phosphorylase b (97.4 kdalton) and carbonic anhydrase (30 kdalton) were fractionated by 10% SDS gel electrophoresis and sub- jected to electrophoretic blotting for 1, 4, 12, and 22 h at 8 Y/cm. The individual protein bands were visualized by radioautography, cut from the nitrocellulose blots, and the radioactivity at each time point determined by liquid scintillation counting.

Fio. 4. Western blot and antibody — IPA detection of intracellular MuLV-specific antigens. Lysates of AKR MuLV-infected E d G2 mouse cells and Moloney MuLV-infected C3H mouse cells were subjected to SDS gel electrophoresis in a l0& polyacrylamide slab gel and the fractionated proteins subsequently trans- ferred to nitrocellulose paper. The blot was then reacted with rabbit antiserum directed against the major inter- nal structural protein (p30) of MuLV and the immune complexes illuminated with IPA followd by radioautog- raphy. Lane A contained lysate equivalent to 3 x 10’ E d G2 cells and lane B to 1 X 10’ C3H cells. Exposure time of this radioautogram was 15 min.

a pH gradient of 6.8—8.8 is set up across the plate electrodes during electrophoresis. However, this does not adversely affect transfer and recirculation of the electrode solution is not necessary. Transfer of pro- teins from isoelectric focusing slab gels (1.5 mm thick) containing urea can be accom- plished in 4—6 h at the same voltage using an electrode solution of 0.7% acetic acid and reversing the electrodes.

Immunodetection of Blotted Proteins

Figure 4 is the Western blot and anti- body—IPA illumination of MuLV-specific products found in AKR virus-infected E S G2 mouse cells and Moloney MuLV-infected C3H mouse cells. The 65,000-dalton poly-

protein precursor (Pr65°°°) of the major MuLV structural proteins (see Ref. (16)) is easily detected in both cell lines with a rab- bit antiserum directed against a constituent of the polyprotein, the viral capsid polypep- tide p30 (17,18). Here the intracellular viral proteins can be seen in a steady-state con-

# dition not fully achievable by metabolic ra- diolabeling. The immunoreactive band identified as P75•°• in Fig. 4 is probably the glycopolyprotein precursor (gPr80°°°) to the glycosylated cell surface antigens gP95/85°°°  (19,20).  Although  P75°o° is  stric- turally and antigenically related to Pr65•°°, it is believed to be a separately initiated translation product of the viral gag gene (21). The lower-molecular-weight polypep- tides in Fig. 4 are specific proteolytic cleav- age products in the processing of Pr65•°• to the individual structural proteins of the virion ( 16).

Although radioiodinated antibodies (both primary and secondary) can be used to de- tect the immobilized antigens, IPA was chosen for the convenience and simplicity of preparing a single, standardizable radio- labeled reagent capable of detectinB mOst types of immune complexes. The varying levels of radiographic “background” seen with different antisera preparations after reaction with IPA is due to nonspecific binding of immunoglobulin to nitrocellu- lose. The signal-to-noise ratio is therefore a function of the relative titers of specific im- mune antibodies and nonimmune IgG in each serum. Various protein solutions and protein admixtures have been used in ef- forts to “block” the nonspecific adsorption of immunoglobulin to the nitrocellulose and thereby increase the signal-to-noise ratio of the IPA-immune complexes. Although none of these mixtures (e.g., ovalbumin, avian globulins, gamma globulin-free bo- vine serum albumin) have duplicated the nitrocellulose binding of mammalian immunoglobulins, solutions of   SP bovine albumin (fraction V) have given satisfactory results and are the most conve- nient and inexpensive. Lengthening or in- creasing the number of washings after either antibody or IPA binding generally had no effect toward decreasing back- ground. Since the Fc portion of rabbit IgG has a greater affinity for protein A than the immunoglobulins of most other animal spe-

# cies (22), one can use much higher dilutions of rabbit antisera compared to the immune sera of other animals (e.g., goat); there is, however, no increase in the signal-to-noise ratio. Most antisera, diluted in Tris—saline— BSA and stored at — 20°C, can be reused many times for reaction with blotted pro- teins. One such antiserum dilution was used more than 40 times over a 10-month period with no apparent diminution in spe- cific immunoreactivity. Freshly radioiodin- ated protein A at 2—5 X 105 cpm/ml was routinely employed to illuminate the im- mune complexes since most results could be seen after 30-60 min of intensifying screen-enhanced radioautography. Smaller inputs of IPA require a consequent increase in radiographic exposure time. Neither the amount nor the specific activity of IPA had any significant effect on the signal-to-noise ratio.

Sensitivity of Immunodetection

# It must be emphasized that sensitivity in Western blotting is primarily a function of the specific antibody titer of the immune serum being utilized. From studies of avian leukosis viruses (see Ref. (23)) it can be cal- culated that as little as 1—2 ng of a specific viral protein (p27) is detectable on blots with the appropriate hyperimmune rabbit serum (1:50 final dilution) exhibiting low nonspecific binding to nitrocellulose. This particular antiserum was capable of precipi- tating approximately I rig of ' zSI-labeled avian myeloblastosis virus p27 at a final di- lution of 1:20,000 in a standard 100-pl ra- dioimmunoprecipitation assay (24). The rabbit antiserum used for the experiments shown in Fig. 4 had a radioimmunoprecipi- tation titer of about 1:12,000 for 0.8 ng of AKR MuLV p30. At a hnal dilution of 1:250 in Western blotting and a radiographic ex- posure time of 15 min after reaction with 2 x HP cpm of IPA/ml, this hyperimmune serum detected the viral polyproteins in ly- sates equivalent to 3 x lo• E6G2 cells

(Fig. 4, lane A) and 1 x 105 C3H cells (lane B). Other experiments with this same anti- serum showed it easily capable of detecting Moloney MuLV antigens from as little as lfP C3H cells.

A.	Ed'G2 Cells

Other Applications of the Western Blot

Figure 5 is the Western blot of the 2-DGE of E d G2 and C3H cell lysates. There is no loss of resolution in the focused spots after transfer and reaction with antibody — IPA. Western blotting used in conjunction with 2-DGE offers high sensitivity, two indepen- dent parameters of fractionation, mainte- nance of point-to-point resolution, simplic-

6	MoDney MuLV infected CMH delis

P75 ’

ity of technique, and the absence of a requirement for costly iit rivo metabolic ra- diolabelings. For these reasons it is an ideal way to characterize complex mixtures of antigens from biopsies of animal tissues.

Electrophoretic transfer coupled with the sensitivity of antibody —IPA illumination makes the Western blot a suitable method for analysis of products synthesized in rirro by messenger-dependent cell lysates (25). Rather than relying upon the incorporation of expensive translation-grade radiolabeled amino acids and the individual immunopre- cipitation reactions for detection of syn- thetic polypeptides, it is possible to visual- ize these products irt situ on a gel blot with specific antibody and IPA. In vitro transla- tions of approximately 1 pg of MuLV ge- nomic RNA resulted in sufficient Pr65 * to be detected after 5—6 h of direct radio- graphic exposure following antibody—IPA treatment of a gel blot; parallel translation with incorporation of ["5S]methionine al- lowed comparable visualization of Pr65=• after overnight exposure (12— 16 h) of a fluorographed gel. Western blotting has also been applied to studies concerned with the segregation of mRNA molecules and subsequent compartmentalized protein synthesis on free and membrane-bound polyribosomes. In this regard, certain nas- cent virus-specific polypeptides associated

Fic. 5. Western blot and antibody -IPA detection of intracellular MuLV-specific antigens fractionated by 2-DGE. Lysates containing either 3 x lfP E dG2 cells or 1 X 105 Moloney-C3H cells were first fractionated by cathode-directed isotachophoresis in cylindrical polyacrylamide —urea gels containing a pH 3.5— 10 am- pholyte. Following isotachophoresis for 30-00 V h, the gels were electrophoresed at right angles in 10Wo polyacrylamide—SDS slab gels. The slab gels were subjected to electrophoretic blotting and the blots sub- sequently reacted with rabbit anti-p30 serum and IPA as described in the text. Radiographic exposure time was 30 min. In both panels the direction of isotacho- phoresis was from right to left. Marker lanes to the right on the second dimension gels show the authentic samples run in a single dimension. A, E S G2 cell ly- sate; B, Moloney-C3H cell lysate.

# with such purified polysome preparations from infected cells have been detected by antibody—IPA illumination on nitrocellu- lose blots.

Problems Encountered in Western Blotting

Distortion due to unequal volume expan- sion of the gel and the nitrocellulose may arise during long-term electrophoresis, par- ticularly at high accelerating voltages. This may lead to diffuse bands (in SDS gel elec- trophoresis) or spots (in 2-DGE). Joule heat- ing can be minimized by keeping voltages below 10 V/cm. Further swelling and shrinking can be prevented by empirically

# matching the methanolic composition of the electrode solution with the amount of crosslinking in the polyacrylamide matrix. Gels prepared from a stock solution of 30% (w/w) acrylamide and 0.8% (w/w) bisac- rylamide undergo no significant volume changes in the electrode solution described. Gels constructed with different matrix ratios may require slight modification in the methanol concentration or short (less than 1 h) preswelling in electrode solution to in- hibit distortion. Failure to have good initial contact between the gel and nitrocellulose can lead to “skewing” of certain bands; this artifact can be seen with the bovine al- bumin and ovalbumin markers in Fig. 2.

A disadvantage of the Western blot from

# SDS-gels is the tendency of antigenically reactive sites (epitopes) on the fractionated molecules to be irreversibly denatured by the detergent (26,27). Although not gen- erally a problem when utilizing polyvalent monospecific antisera for detection, this limitation is quite severe when screening panels of monoclonal antibodies. Since such antibodies are directed against single epitopes within an antigenic molecule (28), there is a likelihood that the reaction of in- terest may be abolished by the dénaturing effect of the detergent. Therefore, when se- lecting monoclonal antibodies for use in Western blotting analysis it is necessary to test the SDS sensitivity of the specific epi- topes. Although it is possible to achieve a greater degree of protein renaturation by substitution of various commercial grades of SDS in the SDS electophoretic electrode buffer ((29,30), unpublished observation), it is best to avoid this detergent altogether if suitable fractionation of sensitive antigens can be accomplished by other gel methods (e.g., slab gel isoelectric focusing).

Unlike blotting to diazotized paper (1,2,6), the radiolabeled probes of Western blotting are not readily removed from non- derivatized nitrocellulose for reuse of the replicas with different probes. Treatment of blots with detergent (5% SDS), chaotropic

agent (3 v NaSCN), or acid (pH 3) fails to preferentially remove probe and back- ground radioactivity relative to the desorp- tion of immobilized antigens.

CONCLUSIONS

The technique of Western blotting should prove valuable for the analysis of proteins fractionated on the basis of molecular weight in SDS gel electrophoresis. Use of unmodified nitrocellulose sheets and an elec- trophoretic mode of transfer provide speed and a simplicity of technique that, combined with essentially complete and quantitative protein transfer, is not achievable by other blotting methods. Coupled with antigen de- tection by antibody and IPA, the Western blot is a very sensitive method for visualizing specific proteins in complex antigenic mix- tures. The use of radioiodinated protein A allows such analyses to be accomplished from cells in culture as well as from necrop- sied animal organs and biopsied tissues with- out having to resort to metabolic radiola- beling of the cellular antigens (N. Burnetts,

R. Tao, and R. Nowinski, unpublished). The blotting method can also be adapted to a large variety of other problems employing SDS gel electrophoresis for analysis. These include, for example, two-dimensional gel fractionation of cellular components, anal- ysis of products synthesized in vitro in re- sponse to exogenous messenger RNA, and studies of nucleic acid binding proteins using appropriate radiolabeled nucleic acid probes (5). Realizing the limitations imposed by SDS denaturation, investigations utilizing monoclonal antibodies for detection may be undertaken if these probes are first selected on the basis of their reactivity against de- tergent-insensitive epitopes.

ACKNOWLEDGMENTS

I wish to thank Dr. R. Nowinski for support and en- couragement in these studies. I also thank Drs. R. Ei- senman, L. Houston, and H. Fan for providing

thoughtful discussion and criticism of the data, and Dr.

R. Tao for invaluable technical assistance.

REFERENCES

1. Southern, E. M. (1975) J. Mol. Biol. 98, 503.

- Alwine, J. C., Kemp, D. J. , and Stark, G. R. (1977) Proc. Nat. Acud. dci. USA 74, 5350.

- Dulaney, J. T., and Touster, 0. (1970) Biochim. Biophys. Acia 196, 29.

- Showe, M. K., Isobe, E., and Onorato, L. (1970)

J. Mol. Biol. 107, 55.

- Bowen, B. , Steinberg, J. , Laemmli, U. K., and Weintraub, H. (1980) Nucleic Acid Res. 8, 1.

- Renart, J., Reiser, J.,  and Stark, G.  R. (1979)

Proc. Nat. Acad. Sci. USA 76, 3116.

- Bittner, M., Kupferer, P., and Morris, C. F. (1980)

Anal. Biochem. 102, 459.

- Towbin, H., Staehelin, T., and Gordon, J. (1979)

Prac. Nat. Acad. Sci. USA 76, 4350.

- Hunter, W. M. (1967) in Handbook of Experimen- tal Immunology (Weir, D. M., ed.), p. 608, Davis, Philadelphia.

t0. O’Farrell, P. H. (1975) J. Biol. Chem. ZSS, 4007.

I t. Laemmli, U. K. (1970) Natune (London} 227, 684.

- Baum, S. G., Horwitz, M. S., and Maizel, J. V., Jr. (1972) 7. Virol. 10, 211.

- Nowinski, R. C., Lostrom, M. E., Tann, M. R., Stone, M. R., and Burnette, W. N. (1979) firm- gy g3, t .

- Schaffner, W., and Weissmann,  C. (1973) Anal.

Biochem. 56, 502.

- Swanstrom, R. , and Shank, P. R. (1978) brief. Bio-

chem. 86, 184.

- Stephenson, J. R., mvare, S. G., and Reynolds,

F. H. (1978) Advan. Cancer Res. 27, 1.

- Bumette, W. N., Holladay, L. A., and Mitchell, W. M. (1976) J. Mol. Biol. 1W, 131.

- Burnette, W. N., and Mitchell, W. M. (1978) J. biro/. 26, 522.

- Tung, I.-S. , Yoshiki, T., and Fleissner, E. (1976)

Cell 9, 573.

- Ledbetter, J., and Nowinski, R. C. (1977) J. Virol.

23, 315.

- Edwards, S. A., and Fan, H. (1979) J. Virol. 30,

551.

- Langone, 1. J. (1978) J. Immunol. Methods 24, 269.

- Eisenman, R., Burnette, W. N. , Heater, P., Zucco,

F., Diggelmann, H., Tsichlis, P., and Coffin, J. (1980) in Biosynthesis, Modification, and Pro- cessing of Cellular and Viral Polyproteins (Koch,

G., and Richter, D. , eds.), p. 233, Academic Press, New York.

- Strand, M., and August, J. T. (1974) J. Virof. 13, 171.

- Pelham, H. R. B. , and Jackson, R. J. (1976) Eur.

J. Biochem. 67, 247.

- Reynolds, J. A. , and Tanford, C. (1970) Proc. Nat. Acad. Sci. USA 66, 1002.

- Waehneldt, T. W. (1975) BioS ystems 6, 176.

- Stone, M. R., and Nowinski, R. C. (1980) Virolog y

UD, 370.

- Swaney, J. B. , Vande Woude, G. F., and Bachrach, H. L. (1974) Aml. Bioc/ierri. 58, 337.

- Lacks, S. A., Springhom, S. S., and Rosenthal,

A. L. 11979) Anal. Biochem. ltD, 357.