# Removal some endocrine-disrupting compounds by N-doped BiOBr  nanoparticle under solar ligth

Delia Teresa Sponza

Dokuz Eylül University, Engineering Faculty, Environmental Engineering Department, Buca-Kaynaklar Campus İzmir Turkey

E mail: delya.sponza@deu.edu.tr

Tel: 0090 05356932709

Some endocrine-disrupting compounds like 17β-estradiol, 17α-ethinylestradiol  and 4-tert-octylphenol was removed from the surface waters using  the N-BiOBr  nanocomposite prepared  under laboratory conditions. N doping on BiOBr improved  the specific surface area and the photocatalytic activity of N-BiOBr. This phenomenon also decreased excessively the acute and  chorinic toxicity originating from the  17β-estradiol, 17α-ethinylestradiol  and 4-tert-octylphenol. The effects of N percentage  removals of the removals of  endocrine-disrupting compounds and activity on   NBiOBr semiconductor  was researched. The effects of N-BiOBr  nanocomposite concentrations (0,5,  1, 1,5 and 2 mg/L), the effects of N percentages ( 2%,4%,6%,10%,15%) the effects of sun ligth power ( 1, 2, 4, 7 and 9 W/m2)  and contacting times (10, 20, 30 and 40 min)  on the of mineralization of endocrine-disruptings were studied.  The toxicities performed by V. Fischeri  and D. magna showed that  the toxicities decreased from 98% to 8%  at N-BiOBr  nanocomposite concentration of  1,5 mg/l,at a N percentage of 10% after a sun ligth power of 4 W/m2  ana afte 30 retention time under sunligth. By doping BiOBr with N an effective  photocatalytic removal process was tedected. Under optimized conditions; 17β-estradiol, 17α-ethinylestradiol  and 4-tert-octylphenol were photocatalically removed with yield as high as 99%, 98% and 97%,respectively.

Keywords : N-doped BiOBr,  photocatalysis, endocrine-disrupting compounds, 17β-estradiol, 17α-ethinylestradiol, 4-tert-octylphenol.