Opto-electronic and thermo-electric properties of Materials based on amorphous thin films

MaryamaHammi*

E-mail: maryama.hammi@um5s.net.ma or mar.hammi@yahoo.com

Abstract

In this study, undoped  lead phosphate glass (PbO-P2O5) and doped  with Cr ,Co, Ni and Zn were synthesized by low cost sol gel technique and deposited on silica glass substrate. To investigate the effect of doping elements  on glass  properties, fillers  doping concentrations  were diversified in the range of 0 wt % - 16wt %, the absorption coefficient was determined using  transmittance data recorded at room temperature ,the optical band gap has also been estimated. Electrical resistivity of the semiconducting thin films were measured as function of fillers concentrations and versus temperature sebsequently. Taking  into account the dc resistivity  depending on the concentration of free carriers and the absorption coefficient, the figure of merit was calculated using Iles and Soclof approach ,it showed a  significant improvement at certain  dopant concentrations . All results were described in detail and they indicated that the films prepared in this study can be used in many nanotechnological applications.

Key words: glass thin films ;  transmition ; absorption coefficient ;  resistivity ; figure of merit