Title: A comparative study on Excised Skins and Human Reconstructed Epidermis with Confocal Raman Spectroscopy and in Microfluidic Diffusion Chambers

Franciska Erdő*

Pázmány Péter Catholic University, Hungary

Abstract

Several ex vivo and in vitro skin models are available in the toolbox of dermatological and cosmetic research. Some of them are widely used in drug penetration testing. The excised skins show higher variability, while the in vitro skins provide more reproducible data. The aim of the current study was to compare the chemical composition of different skin models (excised rat skin, excised human skin and human reconstructed epidermis) by measurement of ceramides, cholesterol, lactate, urea, protein and water at different depths of the tissues. The second goal was to compile a testing system which includes a skin-on-a-chip diffusion setup and a confocal Raman spectroscopy for testing drug diffusion across the skin barrier and accumulation in the tissue models. A hydrophylic drug caffeine and the P-glycoprotein substrate quinidine were used in the study as topical cream formulations. The results indicate that although the transdermal diffusion of quinidine is lower, the skin accumulation was comparable for the two drugs. The various skin models showed different chemical composition. The human skin was abundant in ceramides and cholesterol, while the reconstructed skin contained less water and more urea and protein. Based on these results it can be concluded that skin-on-a-chip and confocal Raman microspectroscopy are suitable for testing drug penetration and distribution at different skin layers within an exposition window. Furthermore, the obese human skins should be treated with caution for skin absorption testing due to its inbalanced composition.

References

- Erdő, F. Skin-on-a-Chip Device for Ex Vivo Monitoring of Transdermal Delivery of Drugs-Design, Fabrication, and Testing. Pharmaceutics 2019, 11, E445, doi:10.3390/pharmaceutics11090445.

- Bajza, Á.; Kocsis, D.; Berezvai, O.; Laki, A.J.; Lukács, B.; Imre, T.; Iván, K.; Szabó, P.; Erdő, F. Verification of P-Glycoprotein Function at the Dermal Barrier in Diffusion Cells and Dynamic “Skin-On-A-Chip” Microfluidic Device. Pharmaceutics 2020, 12, E804, doi:10.3390/pharmaceutics12090804.

- Varga-Medveczky, Z.; Kocsis, D.; Naszlady, M.B.; Fónagy, K.; Erdő, F. Skin-on-a-Chip Technology for Testing Transdermal Drug Delivery—Starting Points and Recent Developments. Pharmaceutics 2021, 13, 1852, doi:10.3390/pharmaceutics13111852.

- Kocsis D, Kichou H, Döme K, Varga-Medveczky Z, Révész Z, Antal I, Erdő F. Structural and Functional Analysis of Excised Skins and Human Reconstructed Epidermis with Confocal Raman Spectroscopy and in Microfluidic Diffusion Chambers. Pharmaceutics. 2022 Aug 13;14(8):1689. doi: 10.3390/pharmaceutics14081689. PMID: 36015315

- Lunter D, Klang V, Kocsis D, Varga-Medveczky Z, Berkó S, Erdő F. Novel aspects of Raman spectroscopy in skin research. Exp Dermatol. 2022 Sep;31(9):1311-1329. doi: 10.1111/exd.14645. Epub 2022 Jul 25. PMID: 3583783

- Kocsis D, Klang V, Schweiger EM, Varga-Medveczky Z, Mihály A, Pongor C, Révész Z, Somogyi Z, Erdő F. Characterization and ex vivo evaluation of excised skin samples as substitutes for human dermal barrier in pharmaceutical and dermatological studies. Skin Res Technol. 2022 Sep;28(5):664-676. doi: 10.1111/srt.13165. Epub 2022 Jun 21. PMID: 35726964

- Kocsis D, Horváth S, Kemény Á, Varga-Medveczky Z, Pongor C, Molnár R, Mihály A, Farkas D, Naszlady BM, Fülöp A, Horváth A, Rózsa B, Pintér E, Gyulai R, Erdő F. Drug Delivery through the Psoriatic Epidermal Barrier-A "Skin-On-A-Chip" Permeability Study and Ex Vivo Optical Imaging. Int J Mol Sci. 2022 Apr 11;23(8):4237. doi: 10.3390/ijms23084237. PMID: 35457056

Biography

Franciska Erdő, PhD graduated from Semmelweis University. She was working for research institutes and pharmaceutical industry in Hungary and Germany. During the last decade she turned to efflux transporters and analysed them in physiological barriers. Since 2014 she has been working for Pázmány Péter Catholic University, Budapest as an associate professor and labhead. Her main interest is drug delivery across the physiological barriers. She is involved in the development of skin-on-a chip microfluidic devices, skin analysis and optical imaging with University of Tours and Semmelweis University. She is the supervisor of BSc, MSc and PhD students and international researchers.