Failure oriented accelerated testing in electronics and photonics materials science and engineering: role, significance, attributes, challenges

E. Suhir

Portland State University, USA

Bell Laboratories, USA (ret)

Abstract

An highly focused and highly cost effective failure-oriented-accelerated-testing (FOAT) was suggested about a decade ago as an experimental basis of the novel probabilistic design for reliability (PDfR) concept. FOAT is supposed to be carried out, when a new technology or a new design are considered and when high operational reliability, like the one required for aerospace, military, or long-haul communication applications, is required. On the other hand, burn-in-testing (BIT) that is routinely conducted at the manufacturing stage of almost any IC product is also of a FOAT type: it is aimed at eliminating the infant mortality portion (IMP) of the bathtub curve (BTC) by getting rid of the low reliability "freaks" prior to shipping the “healthy” products, i.e., those that survived BIT, to the customer(s). When FOAT at the design stage and BIT at the manufacturing stage are conducted, a suitable and physically meaningful constitutive equation, such as, e.g., the multi-parametric Boltzmann-Arrhenius-Zhurkov (BAZ) model, could be employed to predict, from the FOAT data, the probability of failure and the corresponding useful lifetime of the product in the field, and, from the BIT data, as has been recently demonstrated, - the adequate level and duration of the applied stressors, as well as the (relatively low, of course) activation energies of the "freaks". Both types of FOAT are addressed using analytical ("mathematical") predictive modeling, and the general concepts are illustrated by numerical examples. It is concluded that predictive modeling should always be conducted prior to actual testing and that analytical modeling should always complement computer simulations: these two major modeling tools are based on different assumptions, use different calculations techniques, and if the calculated data obtained using these tools are in agreement, then there is a good reason to believe that these data are accurate and trustworthy.  Future work should be focused on the experimental verification of the obtained findings and recommendations.

Biography

Ephraim Suhir is on the faculty of the Portland State University, Portland, OR, USA, and is also CEO of a Small Business Innovative Research (SBIR) ERS Co. in Los Altos, CA, USA, Life Fellow of the Institute of Electrical and Electronics Engineers (IEEE), the American Society of Mechanical Engineers (ASME), the Society of Optical Engineers (SPIE), and the International Microelectronics and Packaging Society  (IMAPS);  Fellow of the American Physical Society (APS), the Institute of Physics (IoP), UK, and the Society of Plastics Engineers (SPE); and Associate Fellow of the American Institute of Aeronautics and Astronautics (AIAA). Ephraim has authored about  500 publications (patents, technical papers, book chapters, books), presented numerous keynote and invited talks worldwide, and received many professional awards, including 1996 Bell Laboratories Distinguished Member of Technical Staff (DMTS) Award for developing effective methods for predicting the reliability of complex structures used in AT&T and Lucent Technologies products, and 2004 ASME Worcester Read Warner Medal for outstanding contributions to the permanent literature of engineering and laying the foundation of a new discipline “Structural Analysis of Electronic Systems”. Ephraim is the third “Russian American”, after S. Timoshenko and I. Sikorsky, who received this prestigious award. His most recent awards are 2019 IEEE Electronic Packaging Society (EPS) Field Award for seminal contributions to mechanical reliability engineering and modeling of electronic and photonic packages and systems and 2019 IMAPS Lifetime Achievement Award for making exceptional, visible, and sustained impact on the microelectronics packaging industry and technology.