An overview of reducing nitrogen oxide emissions in small residential biomass boilers

Mirzaei nezam, PhD, Environmental Health Depatment, Kashan university of Medical Science, Kashan, Iran

Jafarsalehi Morteza ,PhD Student, Environmental Health Depatment, Kashan university of Medical Science, Kashan, Iran

Ebrahimi  Mehdi ,PhD Student, Environmental Health Depatment, Kashan university of Medical Science, Kashan, Iran

Mohammad Mashayekh, Msc in Fluid Mechanic Eng,

Chalous Hospital, Chalous,Mazandaran,Iran

### Abstract

Statement of the Problem: Nearly  one third of the energy produced in the world is consumed in buildings, and it is mainly related to heating and cooling systems. Biomass as an renewable fuel is a suitable alternative to fossil fuels, especially in small domestic and commercial boilers to reduce carbon dioxide emissions and control global warming and climate change. The nitrogen oxides emissions from these boilers has caused environmental and health concerns, especially in dense urban areas.  Methodology & Theoretical Orientation: The aim of the article is to review the method of control the nitrogen oxides emissions, emphasizing the reduction of nitrogen content as the main source of emission in biomass combustion. The effect of planting, holding and harvesting methods of biomass and its storage and processing on nitrogen content, as well as the selection of suitable fuel and boiler and related equipment were investigated and paid attention to.Findings & Conclusion: In this study, it was found that the distance between the trees, the time and frequency of harvesting are the important factors affecting the fuel nitrogen, but excessive use of fertilizer has no effect on it. In addition, the automatic boiler using wood pellets equipped with combustion gas condenser, air staging and lambda sensor is the best type of boiler in the domestic sector to control nitrogen oxide emissions.

Recent Publications

- Cesprini E, Resente G, Causin V, Urso T, Cavalli R , Zaneti M .(2020) Energy recovery of glued wood waste – A review, Fuel ,262,116520.

- Adhikari N, Compostella J, Del Pero C.(2013) Energy and environmental impact of domestic heating in Italy: Evaluation of
national NOx emissions. Energy Policy ,53:353-360.

- Garcia-Maraver A, Zamorano M, Fernandes U, Rabaçal M, Costa M. (2014) Relationship between fuel quality and gaseous and particulate matter emissions in a domestic pellet-fired boiler ,Fuel,119:141-152.

- Mohon Roy  M, Dutta A, Corscadden K .(2013) An experimental study of Combustion and emissions of biomass pellets in a prototype pellet furnace. Applied Energy,108:298-307.

- Khodaei H, Guzzomi F, Patiño  D, Rashidian B, Yeoh G ,(2017) Air staging strategies in biomass combustion-gaseous and particulate emission reduction potentials, Fuel Processing Technology,157:29-41.

Biography

Morteza jafarsalehi is a Ph.D. student at the Environmental health Department. He is working on an review of Biomass heating boiler emissions. Focusing on nitrogen removal from biomass using agronomy and primary measure for NOx control..

Email: Salehi_env@yahoo.com

Presenting author details

Full name: Morteza Jafarsalehi

Contact number:00989113914312

Mail id: linkedin.com/in/morteza-jafarsalehi-77217290/

Category: Oral presentation