# Design and exploration of modified cyclone separator for improved yield of inhalable spray-dried cubosomal powder intended to be used in lung cancer treatment

Akash Deep and Vikas Rana

Punjabi University, India

### Abstract (300 word limit)

This research work aimed to study effect of cyclone design or dimensions on cyclone separator performance in bench-top spray-dryer for the production of inhalable spray-dried cubosomal powder. The different cyclone separators were designed by modifying five geometrical parameters, i.e., length of cyclone body (Lb), length of cyclone cone (Lc), Height (H) and Width (W) of inlet section, and gas/air exit diameter (De) of standard cyclone separator. Further, the designed cyclones were evaluated using Lapple's equations, i.e., number of effective turns (Ne), inlet velocity of air (Vi), pressure drop (∆P) and cut-point diameter (dpc). The modified cyclone (type XVII) with highest Ne, Vi, ∆P and lowest dpc was selected for preparation of powder in comparison to standard cyclone (type I). Further, the Design of experiment (Taguchi design) and Signal to Noise (S/N) ratio were employed to evaluate the effect of formulation parameters (GMO: P407 ratio, PEG200 content, stirring temperature, stirring speed, stirring duration, mannitol: Leucine ratio), spray-drying process parameter (inlet temperature, outlet temperature, aspirator flow rate) and cyclone separator design (type I and XVII) on percentages of device removal efficiency (DRE), aerosolization efficiency (AE), fine particle fraction (FPF) and yield of cubosomal powder. Experimental results indicated that modified cyclone has significantly improved the yield of powder with insignificantly increasing the ∆P. The optimized spray-dried powder possesses sufficient geometrical standard diameter (dG) (3.9 ± 0.25 µm) & mass median aerodynamic diameter (dA) (2.04 ± 0.15 µm) to deliver formulation to deeper lung tissues. It was found that modified cyclone was advantageous over standard one in increasing 1.8 folds yield with DRE, AE and FPF of 94.2%, 52.2%, and 63.8% respectively. Overall, this improvement in powder yield with desirable aerosolization properties will be beneficial in early-stage research and development involving bench-top spray-dryer.

### Image

Recent Publications (minimum 5)

1) Akashdeep Singh, Radhika Sharma, Saurav Chaudhary, and Vikas Rana "Preparation and Characterization of Clopidogrel Bisulfate Hydroxypropyl-β Cyclodextrin Mixed Inclusion Complex for Improved Intestinal Solubility and Anti-Thrombotic Efficacy" Journal of Pharmaceutical Sciences.

2) Akashdeep Singh and Vikas Rana "Exploration of modified cyclone separator for the enhanced recovery of inhalable spray dried cubosomal powder intended to be used for lung delivery" Journal of Drug Delivery Science and Technology, 66 (2021):102848.

3) Akashdeep Singh, Sachin Bhatia and Vikas Rana. "Inhalable Nanostructures for Lung Cancer Treatment: Progress and Challenges" Current Nanomedicine (Formerly: Recent Patents on Nanomedicine), 9 (2019): 4-29.

4) Akashdeep Singh, Bhumika Mangla, Sheshank Sethi, Sunil Kamboj, Radhika Sharma, and Vikas Rana. "QbD based synthesis and characterization of polyacrylamide grafted corn fibre gum." Carbohydrate Polymers 156 (2017): 45-55.

5) Sheshank Sethi, Ranjneet kaur Khurana, Sunil Kamboj, Radhika Sharma, Akashdeep Singh and Vikas Rana. "Investigation the potential of Tamarindus indica pectin-chitosan conjugate for reducing recovery period in TNBS induced colitis" International journal of Biological Macromolecules, 98 (2017): 739-747".

Biography (150 word limit)

Akash Deep is a doctoral student in Pharmaceutical Sciences at Punjabi University, Patiala (Punjab), India, studying under the supervision of Dr Vikas Rana. His PhD research explores the development and evaluation of an inhalable nanostructured-based drug delivery system for poorly water-soluble drugs. Before coming to Patiala, he completed his post-graduation, i.e., M. Pharmacy in Pharmaceutical Sciences, from Punjab Technical University, Jalandhar (Punjab), India. Before that, he received a Bachelor's in Pharmacy in 2011 from Punjab Technical University, Jalandhar (Punjab), India and cleared the GPAT (Graduate Pharmacy Aptitude Test) entrance exam for higher studies. He also had experience in teaching as an Assistant Professor for graduate students. He is particularly interested in the development and evaluation of nanostructured-based DPI's for the effective treatment of pulmonary diseases. He has presented his research work at various national and international conferences on Novel Drug Delivery and has published several research and review papers.