Enhancing thermoelectric properties of conductive polymers using Zr-metal-organic frameworks composite materials

Asmaa Ebrahim, Mohsen Ghali, Ahmed Abdelmoniem

School of Basic and Applied Science, Egypt-Japan University of Science and Technology, New Borg El-Arab City, Alexandria 21934, Egypt

Abstract

Thermoelectric generators (TEGs) that can directly convert waste heat into electricity, have many advantages over traditional electricity generators because they are considered as environment friendly, have no moving parts, have high scalability, which means they can be applied to heat source of any size. The thermoelectric performance of a material is evaluated by the dimentionless figure of merit ZT = α 2 σT/k, where α is the Seebeck coefficient, T is the absolute temperature, σ is the electrical conductivity, and k is the thermal conductivity. So decreasing the thermal conductivity (k) of the material is a critical issue to improve its performance. TEGs are possible devices using for power generation for implantable devices such as Glucose monitor, Drug pump, Cochlear implant and many other devices. The power requirements for most of the implantable devices ranging from 30μW–2mW, which can be provided by TEGs from the human’s body heat.

Polypyrrole, a conducting polymer, attracts most of the attention due to its ease of preparation, high thermal and environmental stability and biocompatibility, which make it a good candidate for implantable TEGs. Most of metal-organic frameworks (MOFs) are considered as insulator with low thermal conductivity and high stability and biocompatibility. MOFs are intrinsically porous that in turn can scatter phonons through the material.

Conclusion and significance, Controlling the polymerization of polypyrrole (Ppy) in presence of Zr-based metal organic-framework (Zr-MOFs) using sodium dodecyl sulphonate (SDS) as a dopant, leads to the formation of a new class of thermoelectric materials based on conducting polymer and highly porous MOFs with enhanced properties for energy production applications. The polymerization of polypyrrole in the Zr-Fumerate pores leads to the formation of homogenously coated MOF-spheres with high crystallinity and a high degree of improvement in many electrical properties such as conductivity and carrier mobility.

References

[1] Rashad, M.M., El-Dissouky, A., Soliman, H.M., Elseman, A.M., Refaat, H.M. and

Ebrahim, A., 2018. Structure evaluation of bismuth telluride (Bi2Te3) nanoparticles with

enhanced Seebeck coefficient and low thermal conductivity. Materials Research

Innovations, 22(6), pp.315-323.

[2] Liang, L., Chen, G. and Guo, C.Y., 2017. Polypyrrole nanostructures and their

thermoelectric performance. Materials Chemistry Frontiers, 1(2), pp.380-386.

[3] Fathieh, F., Kalmutzki, M.J., Kapustin, E.A., Waller, P.J., Yang, J. and Yaghi, O.M.,

2018. Practical water production from desert air. Science advances, 4(6), p.eaat3198.

[4] Jadhav, A., Gupta, K., Ninawe, P. and Ballav, N., 2020. Imparting Multifunctionality by

Utilizing Biporosity in a Zirconium‐Based Metal–Organic Framework. Angewandte

Chemie International Edition, 59(6), pp.2215-2219.

[5] MA, C., SG, P., PR, G. and Shashwati, S., 2011. Synthesis and characterization of

polypyrrole (PPy) thin films. Soft nanoscience letters, 2011.

[6] Gowtham, B., Ponnuswamy, V., Pradeesh, G. et al. Physical Investigations on Various

Weight Percentage of Acetic Acid Doped Polypyrrole by Chemical Oxidative

Polymerization. J Inorg Organomet Polym 30, 2197–2203 (2020).

https://doi.org/10.1007/s10904-019-01408-5

[7] Ke, F., Peng, C., Zhang, T., Zhang, M., Zhou, C., Cai, H., Zhu, J. and Wan, X., 2018.

Fumarate-based metal-organic frameworks as a new platform for highly selective

removal of fluoride from brick tea. Scientific reports, 8(1), pp.1-11.

[8] Zhou, T., Liang, Q., Zhou, X., Luo, H. and Chen, W., 2021. Enhanced removal of toxic

hexavalent chromium from aqueous solution by magnetic Zr-MOF@ polypyrrole:

performance and mechanism. Environmental Science and Pollution Research, pp.1-13