Functional and proliferative effect of hydroxyapatite-germanium dental material in bone marrow mesenchymal stem cells

Jeevithan Elango1*, Artiom Lijnev1, Piedad N. De Aza2, José Manuel Granero Marín1, and José Eduardo Maté Sánchez De Val1

1Department of Biomaterials Engineering, Faculty of Health Sciences, UCAM-Universidad Católica San Antonio de Murcia, Guadalupe, 30107 Murcia, Spain.

2Instituto de Bioingeniería, Universidad Miguel Hernández, Avda. Ferrocarril s/n, Elche,

03202 Alicante, Spain.

Abstract

Hydroxyapatite (HA) is a hard mineral component of mineralized tissues, mainly composed

of calcium and phosphate. Due to its bioavailability, HA is potentially used for the repair and

regeneration of mineralized tissues. For this purpose, the properties of HA are significantly improved by adding natural and synthetic materials. In this sense, the germanium (Ge) mineral was loaded in HA biomaterial by cold isostatic pressure for the first time and characterization and biocompatibility using bone marrow mesenchymal stem cells (BM-MSCs) were investigated. The addition of Ge at 5% improved the solubility (3.32%), stiffness (18.34 MPa), water holding (31.27%) and biodegradation (21.87%) properties of HA, compared to control. Compared to all composite biomaterials, the drug releasing behavior of HA-3% Ge was higher at pH 1 and 3 and the maximum drug release was obtained at pH 7 and 9 with HA-5% Ge biomaterials. Among the different mediums tested, the DMEM-medium showed a higher drug release rate, especially at 60 min. HA-Ge biomaterials showed better protein adhesion and apatite layer formation, which ultimately proves the compatibility in BMMSCs culture. Except for higher concentrations of HA (5 and 10 mg/mL), the different concentrations of Ge and HA and wells coated with 1% of HA-1% Ge had higher BM-MSCs growth than control. All these findings concluded that the fabricated HA biomaterials loaded with Ge could be the potential biomaterial for culturing mammalian cells towards mineralized tissue repair and dental regeneration.

Biography

Jeevithan Elango is affiliated to the Department of Biomaterials Engineering, Faculty of Health Sciences, UCAM-Universidad Católica San Antonio de Murcia, Guadalupe, 30107 Murcia, Spain.