# Optimization	of	anaerobic	co-digestion	of	multiple	feedstocks	for bioenergy recovery- An empirical model application

Anahita Rabii* PhD, Anaergia Inc. Burlington, Ontario & Toronto Metropolitan University, Toronto, Canada

Saad Aldin, Yaser Dahman, and Elsayed Elbeshbishy, PhD, Toronto Metropolitan University, Toronto, Canada

# Abstract

Transition from wastewater treatment facilities as energy consumers to resource recovery facilities with ability to produce energy and value- added products is achievable by optimizing digestion capacity with the existing infrastructure. This study investigated a novel method for optimizing anaerobic digestion of multiple feedstocks for biomethane recovery. A series of experiments were conducted to develop an empirical model for optimizing the mixing ratio based on lipids, proteins, and carbohydrates ratios of the feedstocks as compared to carbon to nitrogen (C:N) or COD to nitrogen ratio (COD:N). The selected feedstocks were real municipal wastes including dairy manure, source separated organic (SSO), and thickened waste activated sludge (TWAS). The experimental data were fitted into the proposed second order polynomial model. The COD:N ratios of TWAS, manure and SSO were 15, 47, and 27 respectively. For the co-digesters, COD:N varied from 19 to 40. The lipids: proteins: carbohydrates ratios were 1:10:4, 1:4:20, and 1:1.6:9 for TWAS, manure, and SSO respectively. Among them SSO had the most ultimate methane production and methane yield corresponding  to  1373  mL  and  332  mL  CH4/g  COD  added.  The minimum ultimate methane production and the methane yield occurred at TWAS mono digestion corresponding to the COD:N ratio of 15 and lipids: proteins: carbohydrates ratio of 1:10:4The results indicated that both minimum ultimate methane and minimum methane yield values occurred at TWAS mono digestion corresponding to the COD:N ratio of 15 and lipids: proteins: carbohydrates ratio of 1:10:4. On the other hand, the maximum ultimate methane and methane yields occurred at the mixing ratios of 2:4:4 corresponding to the COD:N ratio of 28 and lipids: proteins: carbohydrates ratio of 1:3:12 in co-digestion of TWAS/manure/SSO.

# Image

Recent Publications

- R. Hamza, A.Rabii, F. Ezzahraoui, G. Morgan, A review of the state of development of aerobic granular sludge technology over the last 20 years: Full-scale applications and resource recovery, Case Studies in Chemical and Environmental Engineering, 5: 10073, 2022

- A. Rabii, S. Aldin, Y. Dahman, E. Elbeshbishy, “A Review on Anaerobic Co-digestion with Focus on Microbial Consortia Diversity and Digester Mode”, Energies, 12:1106, 2019

- A. Rabii, S. Aldin, Y. Dahman, E. Elbeshbishy, Mesophilic anaerobic co-digestion of manure and thickened waste activated sludge at different mixture ratios, Proceedings of the Water Environment Federation, 4:1938, 2018

- F. Okoye, A. Kankam, T. Oludemi, A. Rabii, E. Elbeshbishy, Effect of free nitrous acid pretreatment on fermentation and anaerobic digestion, Proceedings of the Water Environment Federation, 18:1938, 2018

- A. Rabii, G. Nabi Bidhendi, & N. Mehrdadi, Evaluation of lead and COD removal from lead octoate drier effluent by chemical precipitation, coagulation–flocculation, and potassium persulfate oxidation processes, Desalination and Water Treatment, 43:1-7, 2012

Biography

Dr. Anahita Rabii obtained her PhD in Civil- Environmental Engineering from Toronto Metropolitan University and is focused on waste to energy conversions. Currently her expertise is harnessed with Anaergia Inc. North America, in resource recovery. Dr. Rabii and the engineering group at Anaergia Inc., are developing leading industry innovations by taking waste, and wastewater treatment technologies to a whole level of energy conversions by anaerobic digestion and co-digestion. Dr. Rabii is Chair of Magazine Committee at Water Environment Association of Ontario and is a Sessional Lecturer at George Brown College in Toronto, Ontario, Canada.

Email: anarabii@torntomu.ca

# Presenting author details

Full name: Anahita Rabii

Contact number: +1 (416) 566- 5167 Twitter account: @anahita_rabii

Linked In account: https://www.linkedin.com/in/anahita-rabii Session name/ number: Waste to Energy

Category: Oral presentation