# Examining the effects of fixant solution on the fractographic evidence of fire and impact damaged aerospace composites

Peng Hao Wang, PhD, Purdue University, USA

Natalie Zimmermann, PhD, Purdue University, USA

Keegan Pullen, MS, Purdue University, USA

### Abstract (300 word limit)

The increased popularity and use of composite materials in modern aerospace vehicle structures has led to unprecedented weight savings and performance improvements. However, as the aerospace industry continues to focus on pushing the performance envelope of composite materials, the techniques and protocols pertaining to the failure analysis of aerospace composite materials also need to evolve. One of the scenarios where failure analysis plays a vital role is during an aircraft accident investigation, which is crucial for the continuous improvement of aviation safety. Nonetheless, composites present additional hazards and characteristics that need to be considered at aircraft accident sites, both for safety and evidence preservation. This is especially true when dealing with composites that suffered fire damage from a crash. To prevent the release of potentially toxic gases and fibers from burning composites, and thus to protect the personnel involved, protocols call for the application of a fixant solution at aircraft accident sites where composite materials are present. While fixant solutions are critical to control health hazards, researchers have identified the risk of these chemicals compromising the investigation process by contaminating and masking fractographic features that may be indicators of the failure causes. Therefore, the effect of wetted water, a common type of fixant solution, on fire and impact damaged carbon fiber samples was investigated. To induce the impact damage – and replicate an aircraft accident – a [0]8 carbon fiber sample was impacted in accordance with ASTM standard D7136. After sectioning, scanning electron microscopy (SEM) was used to collect fractographic data and analyze the damage features pre and post fixant application. With the findings of this study, a better understanding of the impact of fixant solutions on the failure analysis process was presented, while simultaneously highlighting the importance of adapting exiting protocols to new generation composite aircraft.
Recent Publications (minimum 5)

- Wang PH, Zimmermann N (2022) Testing composite valve covers for reciprocating engine applications. American Journal of Engineering and Applied Sciences 15:178-188.

- Kim G, Barocio E, Tsutsui W, Wang PH, Dubikovsky SI, Pipes RB, Sterkenburg R (2022) Applicability assessment of thermoset coating onto additively manufactured thermoplastic composite tools. Additive Manufacturing 61:1-10.

- Zimmermann N, Wang PH, Mendonca F, Keller JC (2022) Composite material fires and associated response protocols. Journal of the International Society of Air Safety Investigators 55:22-27.

- Wang PH, Zimmermann N (2021). Maintenance of composite-based aircraft components and structures through the perspective of aviation maintenance technicians in the United States. Collegiate Aviation Review International (CARI) 39:94-133.

- Wang PH, Kim G, Sterkenburg R (2021) An evaluation on the effectiveness of a 3D printed composite compression mold. International Journal of Architectural and Environmental Engineering 15:52-56

Biography (150 word limit)

Peng Hao Wang is currently an Assistant Professor at Purdue University’s Polytechnic Institute, School of Aviation and Transportation Technology. Peng Hao Wang graduated from Purdue University with his bachelor, masters, and PhD degree in 2013. Peng Hao Wang’s research interests focus on composite material life cycle, including the design, testing, manufacturing, repair, and recycling of composite materials. Peng Hao Wang is also a certificated FAA Airframe and Powerplant mechanic with prior experiences working as a line mechanic in the airline industry. Peng Hao Wang’s experiences comprises of airline maintenance, aircraft safety, composite design and testing, composite manufacturing, composite repair, and composite recycling.

Email: pwang@purdue.edu

Notes/Comments: N/A

Presenting author details 
Full name: Peng Hao Wang
Contact number: +1-765-714-4603
Twitter account: N/A
Linked In account: https://www.linkedin.com/in/peng-hao-wang-89a9b9192

Session name/ number: Track 11 Composite Materials
Category: Oral presentation