Evaluation of Health Implication of Radiological Dose Indices in a Mineralized Zone Environment, Southern Nigeria

Olalekan Olatunji*

Keele University, United Kingdom

Abstract

Exposure and radioactivity level of radionuclides in a mineralized zone were investigated on a view to evaluate its radiological health impact in the environment. An advanced Gamma scout survey meter was used to determine the exposure level while a gamma ray spectroscopy (Sodium Iodide detector NaI (TI) determine radionuclides. Some of the radiological indices that were evaluated is the absorbed dose, effective dose, annual equivalent dose, Internal and External hazard indices, annual gonadal dose equivalent, gamma index and the Excess lifetime cancer risk.

A digital survey meter 992 Fluke Victoreen Gamma Scout Model detector was used to determine the exposure rate in the vicinity of the sampling point. The gamma-scout that was used to determine the exposure level was oriented vertically upward through the survey point. In the whole sampling area, ten different measurements at each site were activities of petroleum drilling, coal mining and bitumen exploration were conducted. The data measured were read directly from the display screen of the gamma-scout. The gamma scout was placed 1 m above the soil level for an hour which enabled us to determine the average exposure in each of the sampling points.

Environmental media (Soil, water and foodstuff) samples were collected in order to determine the radionuclides such as potassium-40, thorium 232 and uranium-238 in the laboratory. All the samples were weighed for the soil (50 g), water (100 ml) and plants (50 g) respectively and put in a well labelled cylindrical airtight plastic container previously washed thoroughly with dilute HNO3 and rinsed with distilled water. Instrumentation analysis was carried out at centre for Energy and research development (CERD), Obafemi Awolowo University, Ile-Ife, Nigeria.

The preliminary results obtained for the exposure level in petroleum drilling environment for ranges from 0.0825 to 0.1134 μSv/hr with a mean value of 0.0945 μSv/hr, while in the bitumen exploration and exploitation area ranges from 0.0870 to 0.1230 μSv/hr with a mean value of 0.1101 μSv/hr and the coal mining environment ranges from 0.075 to 0.1500 μSv/hr with a mean value of 0.1066 μSv/hr respectively. Results obtained were compared to the data obtained by the international Atomic Energy Agency (IAEA), (2000) recommended standard.

The result in this study will provide insight into environmental behaviour and fate of contaminant and serve as baseline data for future investigation and monitoring.

References:

Abdel-Ghani, N.T., Elchaghaby, G.A., (2007). Influence of operating conditions on the removal of Cu, Zn, Cd and Pb ions from wastewater by adsorption. Int. J. Environ.  Sci. Technol. 4, 451–456.

Adedokun, T. A. 1982. The hydrogeology of the Okitipupa tar sand area. M. Sc. Thesis 145p.

Department of Geology, University of Ife, Nigeria.

Adegoke, O. S. 1980. Geotechnical investigations of the Ondo State bituminous sands: VoL I. Geology and Reserves Estimates. 257p. Geological Consultancy Unit, Department of Geology, University of Ife, Nigeria

Adegoke O. S. and Omatsola M. E. (1981). Tectonic Evolution and Cretaceous Stratigraphy

of the Dahomey Basin. J. Min. Geol. 18 (1): 130-137.

Ademola A.K, Olaoye M.A, Abodunrin P.O (2015). Radiological safety assessment and determination of heavy metals in soil samples from some waste dumpsites in Lagos and ogun state, south-western, Nigeria. Journal of Radiation Research and Applied Sciences.2015;8(1):148153.https://dx.doi.org/10.1016/j.jrras.2014.12.010.doi:10.1016/j.jrras.2014.12.010.

Aditi, C. (2010). Disequilibrium of Naturally Occurring Radionuclides and Distribution of Trace Elements in a Highly Mineralised Zone. Ph.D thesis Homi Bhabha National Institute.

Ahmed, M. K., Baki, M.A., Islam, M.S., Kundu, G.K., Sarkar, S.K., Hossain, M.M., (2015a). Human health risk assessment of heavy metals in tropical fish and shell fish collected from the river Buriganga, Bangladesh. Environ. Sci. Pollut. Res., http://dx.doi.org/10.1007/s11356-015-4813-z

Biography:

Olalekan OLATUNJI is a third year PhD Student in school of Geography, Geology and Environment in Keele University, United Kingdom. A lecturer and a researcher in Obafemi Awolowo University focused on Health and Environmental Physics. Olalekan have presented and published papers in the area of radioecology, environmental sustainability and health physics. Olalekan is a member of International Organization of medical physicist, Nigeria Institute of Physics, UK geologist society and Institute of Environmental sustainability.