# Experimental study on sildenafil citrate drug solubility in supercritical carbon dioxide

Gholamhossein Sodeifian,

University of Kashan, Kashan, Iran

### Abstract

Statement of the problem: According to the sexual dysfunction definitions [1], erectile dysfunction is the persistent inability to get and maintain an erection for satisfactory sexual performance. Among available methods for treatment, oral medications are the most common. In this regard, sildenafil citrate belongs to a class of medications called phosphodiesterase type 5 (PDE 5) inhibitors, which can restore the natural process of erection. Despite rapid absorption after administration, absolute bioavailability of sildenafil citrate has been reported about 40% [2]. One of solutions to improve the bioavailability of this medicine is production of nanoparticles via supercritical fluid-based technologies [3]. To follow these technologies, knowledge of solubility of sildenafil citrate in the supercritical fluid at different temperatures and pressures, is the first step. However, experimental performances at various operating conditions are usually impractical. Therefore, application of an acceptable theoretical approach such as semi-empirical models [4] would be found beneficial.

Methodology and theoretical orientation: This study is aimed to measure the solubility of sildenafil citrate in supercritical carbon dioxide (SCCO2) at equilibrium conditions of 308-338 K and 120-270 bar. Afterwards, Sodeifian’s model [5] as a semi-empirical model was applied to correlate the obtained solubility data. Using the function of absolute average relative deviation (AARD%), a statistical criteria, the correlating ability of this model was evaluated. More information on procedure and formulas are available elsewhere [6].

Findings: The minimum and maximum solubility data, in terms of equilibrium mole fraction were 0.14×106 and 5.91×106, respectively. With respect to the obtained experimental data, crossover pressure point was observed near 230 bar. Besides, Sodeifian’s model showed an acceptable ability for correlating the data with an AARD% of10.49%. The correlative constants of the model for sildenafil citrate are presented below:

| a0 | a1 | a2 | a3 | a4 | a5 |
| --- | --- | --- | --- | --- | --- |
| -3.06 | 0.075 | 0.008 | 0.002 | -0.012 | -961.4 |

Conclusion & significance: The results of this study indicated that sodeifian’s model has a good performance for correlating the solubility data which makes it appropriate for further applications in supercritical carbon dioxide-based technologies such as gas anti-solvent (GAS).

Experimental data for sildenafil citrate solubility in SCCO2

Experimental data for sildenafil citrate solubility in SCCO2

Recent Publications

- McCabe MP, et al. (2016) Definitions of sexual dysfunctions in women and men: a consensus statement from the Fourth International Consultation on Sexual Medicine 2015. The journal of sexual medicine 13: 135-143.

- Pirhayati, FH, et al. (2017) Thermodynamic solubility and density of sildenafil citrate in ethanol and water mixtures: Measurement and correlation at various temperatures. Journal of Molecular Liquids 225: 631-635.

- Islam T, et al. (2022) Development of nanoparticles for pharmaceutical preparations using supercritical techniques. Chemical Engineering Communications 209: 1642-1663.

- Sodeifian G, Sajadian SA, Derakhsheshpour  R (2022) CO2 utilization as a supercritical solvent and supercritical antisolvent in production of sertraline hydrochloride nanoparticles. Journal of CO2 Utilization, 55: 101799.

- Sodeifian G, Razmimanesh F, Sajadian SA (2019) Solubility measurement of a chemotherapeutic agent (Imatinib mesylate) in supercritical carbon dioxide: Assessment of new empirical model. The Journal of Supercritical Fluids 146: 89-99.

- Sodeifian G, Alwi RS, Razmimanesh F, Abadian M (2022) Solubility of Dasatinib monohydrate (anticancer drug) in supercritical CO2: Experimental and thermodynamic modeling. Journal of Molecular Liquids 346: 117899.

Prof. Dr. Gholamhossein Sodeifian (1971) graduated in chemical engineering (M.S) from University of Tehran, in 1997 and received his doctorate (Ph.D), in polymer engineering from Tarbiat Modares University, Tehran, in 2002. Prof. Sodeifian is currently academic member of Chemical Engineering Department of University of Kashan. His research group focused on extraction of essential and seed oils, solubility measurement of solid medicines, and micro- and nanoparticle formation of pharmaceutical materials in supercritical carbon dioxide (SCCO2) via various methods. He has also developed, for the first time in the world, a new and efficient technique for nanoparticle formation, i.e., ultrasonic assisted rapid expansion of supercritical solution into a liquid solvent (US-RESOLV). He has published more than 83 ISI scientific papers and several books. Furthermore, he has developed Sodeifian’s model for drugs solubility. Prof. Sodeifian has been assigned and included in the worlds’ top 2% of scientists list in 2021 and 2022.

Email: sodeifian@kashanu.ac.ir

Presenting author details

Full name: Gholamhossein Sodeifian

Contact number: +98213155913393

Mail id: sodeifian@kashanu.ac.ir

Category: Oral presentation

Presenting author details

Full name:

Contact number:

Mail id:

Category: (Oral presentation/ Poster presentation)