# Production of hydrogen by biomethane microwave plasmalysis: a CO2 negative process with impact on the energy transition

Damien DUSSOL1

Alvaro MARTIN ORTEGA1 Arnaud BOUTIBONNES1

Marilena RADOIU2

1Sakowin SAS, Aix-en-Provence, France

2Microwave Technologies Consulting SAS, Lyon, France

# Abstract

At present our society is facing two related challenges: depletion of hydrocarbon fuel resources and the worrying prospects of global climate change associated with enormous volumes of fossil-derived CO2 emissions. Currently the global predominant energy sources are fossil- based, i.e. coal, oil, natural gas, and their usage results in the release of

∼32 billion metric tons of CO2/year in the atmosphere [1]. As such, the development of new alternative ways to produce and store energy as well as new industrial technologies must be based on environmentally responsible solutions. One of the best options to produce carbon-free energy is the decarbonization of fuel energy [2]. In this perspective, it is expected that hydrogen (H2) will play an important role in the world’s future [3]. Hydrogen, one of the most abundant elements on Earth, has the potential to dramatically reduce our dependence on fossil fuels. A potential source of H2 is biomethane also known as ‘renewable natural gas’, a gas with high potential of reducing the release of greenhouse gases [4]. Biomethane can be decomposed into H2 and solid carbon. Microwave sustained plasma appears to be an excellent candidate for the activation of the thermodynamically stable methane molecule, as it is an excellent power transfer medium (electrons to molecules) [5] and is compatible with sustainable electricity. This work concerns the characterization of methane discharges in a 2.45 GHz microwave reactor sustained at atmospheric pressure for the development of an efficient source for H2 production. The experimental protocol takes into consideration various gas mixture constituents and operating conditions, e.g., flow rate, microwave input power, as to optimize the decomposition of methane molecule into H2 and solid carbon. The economics of microwave-assisted plasma methane decomposition as a hydrogen source are evaluated.

# Image

Recent Publications

- T.A. Boden, G. Marland, R.J. Andres. 2012 Global, Regional, and National Fossil-Fuel CO2 Emissions (Oak Ridge, TN: Carbon Dioxide Information Analysis Center, Oak Ridge National Laboratory, US Department of Energy), online at https://cdiac.ess- dive.lbl.gov/trends/emis/overview.

- N.Z. Muradov, T.N. Veziroglu. 2008. “Green” path from fossil- based to hydrogen economy: An overview of carbon-neutral technologies. Int. J. Hydrogen Energy 33 6804–39.

- Fuel Cell and Hydrogen: Hydrogen Roadmap Europe: a Sustainable pathway for the European energy transition, on-line at https://www.fch.europa.eu/publications/hydrogen-roadmap- europe-sustainable-pathway-european-energy-transition.

- R. Singh, P. K. Mishra. et al. Bioenergy Research: Evaluating Strategies for Commercialization and Sustainability Chapter 11 (2021).

- Inayat, A., Tariq, R., Khan, Z. et al. A comprehensive review on advanced thermochemical processes for bio-hydrogen production via microwave and plasma technologies. Biomass Conv. Bioref. (2020).

Biography

Dr. Damien DUSSOL has been passionate about energies since his Master in Chemistry and Chemical Engineering in 2016. That led him naturally to the French Institute of Petroleum and New Energies where he obtained his Ph.D. with a thesis related to the transformation of ethanol into butadiene, a key compound of the Oil&Gas industry, traditionally produced from petroleum. After a postdoc studying the Haber-Bosch process in a BASF daughter company, he started to work as a chemistry specialist at Sakowin Green Energy, France, a pioneer of the energy transition.

Email: damien@sakowin.com

Notes/Comments:

# Presenting author details

Full name: DUSSOL Damien

Contact number: +33 6 19 64 51 64 Twitter account:

Linked In account: linkedin.com/in/damiendussol Session name/ number:

Category: (Oral presentation/ Poster presentation)