Enhanced Performance of WO3/SnO2 Nanocomposite Electrodes with Redox-Active Electrolytes for Supercapacitors

Tamiru Deressa morka

National Taiwan University of Science and Technology, Taiwan

Abstract

For effective supercapacitors, we developed a process involving chemical bath deposition, followed by electrochemical deposition and calcination, to produce WO3/SnO2 nanocomposite electrodes. In aqueous solutions, the hexagonal WO3 microspheres were first chemically deposited on a carbon cloth, and then tin oxides were uniformly electrodeposited. The synthesized WO3/SnO2 nanocomposite was characterized by XRD, XPS, SEM, and EDX techniques. Electrochemical properties of the WO3/SnO2 nanocomposite were analyzed by cyclic voltammetry, galvanostatic charge‒discharge tests, and electrochemical impedance spectroscopy in an aqueous solution of Na2SO4 with/without the redox-active electrolyte K3Fe(CN)6. K3Fe(CN)6 exhibited a synergetic effect on the electrochemical performance of the WO3/SnO2 nanocomposite electrode, with a specific capacitance of 640 F/g at a scan rate of 5 mV/s, while that without K3Fe(CN)6 was 530 F/g. The WO3/SnO2 nanocomposite catalyzed the redox reactions of [Fe(CN)6]3/[Fe(CN)6]4− ions, and the [Fe(CN)6]3−/[Fe(CN)6]4− ions also promoted redox reactions of the WO3/SnO2 nanocomposite. A symmetrical configuration of the nanocomposite electrodes provided good cycling stability (coulombic efficiency of 99.6% over 2000 cycles) and satisfied both energy density (60 W h kg−1) and power density (540 W kg−1) requirements. Thus, the WO3/SnO2 nanocomposite prepared by this simple process is a promising component for a hybrid Pseudocapacitor system with a redox-flow battery mechanism.

Keywords: supercapacitor; nanocomposite; energy storage; redox-additive; tungsten oxide; tin oxide

Biography

Mr Tamiru Deressa Morka is a PhD candidate at National Taiwan university of Science and Technology. He received his M.Sc. degree in Quantum optics and Information in 2019 from Jimma University and B.Sc. degree in applied Physics in 2012 from Jimma University, Ethiopia. His research focus on Energy storage Materials and Supercapacitor application.