Modified nonlocal boundary conditions for asymmetric vibrations of functionally graded annular nanoplates under thermal environment

Rahul Saini

H N B Garhwal Central University, India

# Abstract

Statement of the Problem: The Eringen’s nonlocal theory shows paradoxes for free boundary conditions while studying the vibration, thermomechanical and bending behavior of nanobeams and nanoplates. To avoid this issue, the modified boundary conditions are developed for the free asymmetric vibrations of functionally graded (FG) annular nanoplates under thermal environment. Hamilton’s principle is used to obtain the governing equations on the basis of first- order shear deformation theory together with Eringen’s nonlocal elasticity theory. The quadrature method along with the Chebyshev collocation method were adopted. The choice of method is based on the rate of convergence of the method and then employ the same to obtain the numerical values of frequencies. The effect of various parameters together with nonlocal boundary conditions on the nondimensional frequencies was studied. The validation of results with those available in the literature is presented to validate the accuracy of the results and the efficiency of the authors’ technique.

# Recent Publications

- Saini R, Pradyumna S (2023) Asymmetric vibrations of functionally graded annular nanoplates under thermal environment using nonlocal elasticity theory with modified nonlocal boundary conditions. Journal of Engineering Mechanics (ASCE) 149(5): 04023022-1-16.

- Ahlawat N, Saini R (2023) Vibration and buckling analysis of elastically supported bi-directional FGM Mindlin circular plates having variable thickness. Journal of Vibration Engineering & Technologies.

- Saini R, Saini R, Kumar A, Khadimallah MA (2022) Free axisymmetric vibrations of heated non-uniform bi-directional FGM Mindlin rings employing quadrature approaches. Thin- Walled Structures 184:110482.

- Saini R, Ahlawat N, Rai P, Khadimallah MA (2022) Thermal stability analysis of functionally graded non-uniform asymmetric circular and annular nano discs: Size-dependent regularity and boundary conditions. European Journal of Mechanics / A Solids 94:104607.

- Saini R, Pradyumna S (2022) Effect of thermal environment on the asymmetric vibration of temperature-dependent two-dimensional functionally graded annular plate by Chebyshev polynomials. Journal of Thermal Stresses 45(9):740-761.

# Biography

Dr. Rahul Saini has his expertise in vibration, buckling, and bending analysis of beams, plates, nanobeams and nanoplates made of functionally graded materials (ceramic + metal). He has developed the codes of algorithms for numerical techniques used to solve the modelled differential equations. He has derived the modified boundary conditions to avoid the paradoxes in the nanoplates with free boundary conditions. He has built this model after years of experience in research, evaluation, teaching and administration in education institutions.