Bioethanol production on cheese whey using an Algerian yeast strain isolate

Khaled Boudjema, Fethia Fazouane-Naimi and Amina Hellal

Université de M’Hamed Bougara, Algérie

Ecole Nationale Polytechnique,  Algérie

Abstract

The bioethanol is considered as an important renewable energy source for future. In this study, cheese whey, a dairy industry waste, was utilized as substrate to produce bioethanol using an Algerian yeast strain Saccharomyces cerevisiae DIV13-Z087C0VS. Before starting the alcoholic fermentation, lactose, main sugar of whey, has been hydrolyzed enzymatically with β-galactosidase to obtain fermentescible sugars glucose and galactose. Then, the batch fermentation was conducted under the following operating parameters such as temperature 28.38°C, pH 4.31 and yeast extract supplement 0.4 % (w/v).

The results of physicochemical and biochemical analyzes of whey showed on the one hand its richness to the nutrient compounds; protein (3.56 ± 0.02 g/l), lactose (58.50 ± 0.50 g/L), total nitrogen (g/l) (0,575 ± 0,025) and mineral salts (Na = 0.734 ± 0.014, Ca =1.590 ± 0.005, K = 0.082 ± 0.002, Zn = 0.001 ± 0.000, Mg = 0.186 ± 0.006, P = 0.858 ± 0.008, Cl =1.611 ± 0.090) and on the other hand , it is characterized with high values of Biological oxygen demand (BOD5)  and Chemical  oxygen demand  (COD) which are  48,55 ± 0,45 g d’O2/l and 73 ± 0,60  g d’O2/l).

Under optimized conditions, the batch fermentation gave the highest ethanol concentration of 18.31 g/l with highest values of volatile compounds such as acetaldehyde 10.51mg/l, propanol 10.76 mg/l, 2-methyl-1-butanol 23.17 mg/l, 3- -methyl-1-butanol 41.94 mg/l, 2- methyl- 1-propanol 31.81 mg/l and ethyl acetate 2.24 mg/l. Furthermore, the production of bioethanol on same conditions on cheese whey powder (CWP) of 20 % (w/v) gave a concentration of 64 g/l. This finding demonstrated that cheese whey can be used as a sustainable feedstock for biorefineries.

Recent Publications

- De Glutz FN (2009) Fuel bioethanol production from whey permeates. Phd Thesis, École polytechnique fédérale de Lausanne Suisse. p. 179

- Dos Santos LF, Alvares da Silva Campos TC, Suguimoto HH (2014) Bioethanol fermentation of cheese whey powder (cwp) solution using unexplored strain of kluyveromyces marxianus cct 7585. The Experiment; 27(1):1852–62.

- Lin Y, Zhang W, Li C (2012) Sakakibara K, Tanaka S, Kong H. Factors affecting ethanol fermentation using Saccharomyces cerevisiae BY4742. Biomass and Bioenergy. p. 47

- De Moura Ferreira, M. A., da Silveira, F. A., and da Silveira, W. B. (2022). Ethanol stress responses in Kluyveromyces marxianus: Current knowledge and perspectives. Appl. Microbiol. Biotechnol. 106, 1341–1353

- Cruz, M.L.; de Resende, M.M.; Ribeiro, E.J (2021) Improvement of Ethanol Production in Fed-Batch Fermentation Using a Mixture of Sugarcane Juice and Molasse under Very High-Gravity Conditions. Bioprocess Biosyst. Eng. , 44, 617–625

Biography

Khaled Boudjema is an assistant-teacher in department of Biology and a researcher in research laboratory of food technology at the Université de M’Hamed Bougara, Algérie.