Kheemp(Leptadenia pyrotechnica)-Biomass:A new potential

source of Bioenergy from hot arid Thar Desert.

ABSTRACT.

Kheemp ( Leptadenia pyrotechnica)  a latex bearing medicinally important fibre yielding plant of Asclepiadaceae  with significant oil content in its seed ( 30%-32 % ) may be developed as a potential bio energy resource. Huge quantum of research work has been done on its medicinal, fibre application and ecological aspects, but very scanty scientific information about its potential as bio fuel is available on the internet. To best of our knowledge. We have analysed for the first time, some important fuel properties of Kheemp ripe pods parts viz. seed ,empty pod wall , whole pod and residual ligno-cellulosic stem biomass ( after fibre extraction. ) and  compared the results with corresponding values of the bio energy plants currently being analysed. Preliminary results of our experiments showed higher  gross calorific value of Petroleum ether extracts (PEE) of different pod parts which ranged from 10716-11291 cal/gm, to non-edible oilseed ( 8700-9900 cal/gm.) and similar to  latex bearing plants and petroleum fuels.( above 10000 cal/gm.) , low or similar condarson carbon residues( 0.29 % -0.820% ) to bio  energy plants( 0.40% - 0.64%) and  low Kinematic viscosity at 40 deg.C ( 2.04 ( cst)- -9.50 ( cst)  to bio energy plants (11%--39.6%). The residual stem has high gross calorific value of (4400 cal/gr). Fixed carbon (20.55 %), volatile matter (73.56 %), ash- (5.89%) and Moisture (7.12%) with favourable handling physical parameters. Our Initial results indicate that Kheemp ripe pod parts have potential to be developed as source of liquid bio fuel. Residual stem biomass as solid biofuel   or it may be used to generate bio electricity after gasification or for liquid biofuel production after pyrolysis. Further exhaustive studies on fuel properties and conservation and propagation of Kheemp is required.