Industrial production of  bioethanol and biomaterials from

lignocellulose in China

Run-Cang Sun, Center for Lignocellulose Chemistry and Materials, Dalian Polytechnic University, Dalian, China

Abstract

An environmentally friendly industrial biorefinery of the lignocelluloses such as corn cob and other cereal straws for production of bioethanol and biomaterials by a combination of hydrothermal pre-treatment and alkali post-treatment will be reported. 30 thousands tonnes of bioethanol , 12 thousands tonnes of oligosaccharides and 10 thousands tonnes of xylitol with a purity of more than 97%, 3 hudrend tonnes of arabiose with a purity of more than 98.5%, and 10 thousands tonnes of lignin with a purity of more than 94% have being produced from 200 thousands of corn cob per year at Shandong Longlive Bio-Technology Co., Ltd, China. The recovered lignin, which is a significant source of CO2 emissions if burned, was activated under alkaline conditions and then used to produce lignin-phenol-formaldehyde (LPF) adhesives with a yield of about 50 housands tonnes per year for partially replacing the expensive phenols (50%) in the commercial production of biocomposite boards for construction and furniture. Finally, the cellulose-rich fraction, which has a large surface area and total pore volume, is enzymatically hydrolyzed and then fermented into bioethanol with a high conversion, in which 3 tonnes of the cellulose-rich fraction can produce one ton of bioethanol. These value-added hemicelluloses- and lignin-derived products have greatly improved the economy of both lignocellulose conversion and bioethanol production. Another industrial utilization of the by-products of lignins from the bioethanol production as rubbish bags used in shanghai city and agricultural mulching films used on farms as well as other lignin based on biomaterials will be also reported.

Recent Publications

- Sun SN, Chen X, Tao YH, Cao XF, Li MF, Wen JL, Nie SX, Sun RC (2019) Pretreatment of Eucalyptus urophylla in gamma-valerolactone/dilute acid system for removal of non-cellulosic components and acceleration of enzymatic hydrolysis. Industrial Crops and Products 132: 21-28.

- Wang HM, Sun YC, Wang B, Sun D, Shi QT, Zheng L, Wang SF, Liu SJ, Xia RR, Sun RC (2019) Insights into the Structural Changes and Potentials of Lignin from Bagasse during the Integrated Delignification Process. ACS Sustainable Chemistry & Engineering 7: 13886-13897.

- Xu JY, Xia RR, Zheng L, Yuan TQ, Sun RC (2019) Use of xylooligosaccharides (XOS) in hemicelluloses/chitosan-based films reinforced by cellulose nanofiber: Effect on physicochemical properties. Food Chemistry 298: 125041.

Biography

Professor Dr. Run-Cang Sun is the director of Center for Lignocellulose Chemistry and Materials, Dalian Polytechnic University, Dalian, China. He has being focused on the biomaterials, biofuels, and chemicals from lignocelluloses. He has published more than 850 SCI papers in peer reviewed international journals in English and 50 papers in international conferences as plenary, or keynote reports, which have been totally cited over 37,000 citations (his Google Scholar h-index of 93). He edited a book under the title of “Cereal Straw as a Resource for Sustainable Biomaterials and Biofuels: Chemistry, Extractives, Lignins, Hemicelluloses and Cellulose” 2009, Elsevier, London. As invited author, he has published 35 book chapters (in English) such as Encyclopaedia of Separation Science (Academic Press, London). He has got 9 items of research awards and 121 patents. Prof. Sun is a Fellow of The Royal Society of Chemistry, UK and Editor-in-Chief of “Industrial Crops and Products”,   Associate Editor of “Carbohydrate Polymers”，“BioResources” Journal, and “Journal of Biobased Materials and Bioenergy”, He is also a member of Editorial Board in “Chemsuschem”,“Journal of Agricultural and Food Chemistry”, “International Journal of Cellulose Chemistry and Technology” et al. In March 2020, Professor Run-Cang Sun was awarded 2020 Anselme Payen Award.