Photogalvanic insolation device for accumulation, conversion & utilization solar radiation through photoimgeable dyes Fast green – Toludine blue – EDTA.

Bijendra Singh1 and Dr.Chhagan Lal2

1School of Chemical Sciences, Central University of Gujarat, Gandhinagar-382030

2Department of Chemistry, Harcourt Butler Technical University, Kanpur-208002

Email: c.lal9940@gmail.com, bijendrasingh21@gmail.com, WhatsApp No.8802090904 .

Abstract:

Photogalvanic cells are designed based on the photogalvanic effect; On acquiring energy, conversion solar energy into electricity and its storage. Production of potential between two electrodes respectively illuminated and dark chamber in H -shape cell through the influence of light on the electrode potential arose due to a photochemical process in the body of the electrolyte. System FG-TB-EDTA designed for conversion solar energy into chemical energy. Photogalvanic cell based on the excitation of electron and circulation in system  , in which the photogalvanic cell has potential to revolutionize the existing solar cells due to its low cost in market and inherent storage capacity. The main purpose of the paper is the results of research published related to this phenomenon and to propose the mechanism for the photocurrent generation. Photoimgeble dye Fast green, Toludine blue and reductant EDTA used for developed a new insolation and conversing system in this system EDTA playing role as a reducing agent for photoigeble dyes  FG & TB in alkaline medium. Designed new system based on photochemical conversion for  accumulated insolation and utilized of sun radiation through FG-TB-EDTA. Effect of different parameter dye, rductant ,pH,light intensity,area of Pt electrode-v ,diffusion length was studied on this new system for electrical output. The photogalvanic performance for insolation is determined for this device shown CE,FF & storage capacity .Photopotential & Photocurrent generated by this system were 980.0mV & 116.0µA respectively.Photogalvanic device can be utilize in dark till 34 min.

Key Words:-Conversion efficiency,power poit,fill factor,Cell performance,Fast Green ,Toludine blue.

References:

E. Rabinowitch, J Chem. Phys. 8 (1940) 560.

Lua, L., Renjie, L., Fana K. and Tianyou, P., (2010) Effects of annealing conditions on the photo-electrochemical properties of dye-sensitized solar cells made with ZnO nanoparticles,Solar Energy, Vol84, Issue 5, pp844-853.

S. Zhang, A. Islam, X. Yang, C. Qin, K.  Zhang, Y. Numata, H. Chen, L. Han, J. Mater. Chem. A.  1(2013) 4812.

Lal , M.,  and  Gangotri , K. M. ,  “The optimum conversion efficiency in nile blue arabinose system by photogalvanic cell”,  Advances in Engery Research , Vol.3 ,  No.3 ,  PP.143-155 ,  2015

Yadav K Sushil ,(2017),Photogalvanic effect by using natural dye : Rose flower extract –Ascorbic acid system ,Int.J Engineering Technology science and research ,Vol(070,issues-4,PP-509-513.

Biography

Bijendra Singh is a research scholar in School of Chemical Sciences, Central University of Gujarat,India.Mr. Singh research field in Chemistry – Photochemistry, Solar Energy, Nano-Science and Technology, Organic Chemistry.

Email: bijendrasingh21@gmail.com

LIST OF PUBLICATIONS:

- Bijendra Singh,Dr.Chhagan Lal, “Enhanced efficiency of Photogalvanic cell based on mixed Triphrnylmethane dyes” , Assian Journal of Chemistry, (2020),Vol-32,No.2,466-470. https://doi.org/10.14233/ajchem.2020.22547

- Bijendra Singh,Dr.Chhagan Lal,Nakul Kumar , “Utilization of mixed Naphthol Green B and Janus Green as photosensitisers in Photogalvanic cell for solar Energy conversion and storage”, Assian Journal of Chemistry, Asian Journal of Chemistry; Vol. 32, No. 8 (2020), 1914-1920 https://doi.org/10.14233/ajchem.2020.22712.

- BIJENDRA SINGH , CHHAGAN LAL, NAKUL KUMAR  and ANGIK K. PATEL, “Synthesis and Biological Activities of Some Novel Spiro Heterocyclic Pyrrolizidine Derivatives of 11H-indeno[1,2-b]quinoxaline through 1,3-Dipolar Cycloaddition” Asian Journal of Chemistry; Vol. 32, No. 5 (2020), 1255-1258, https://doi.org/10.14233/ajchem.2020.22630

- Bijendra Singh, Anil Kumar and Kapil Tyagi, “ Physiochemical Analysis of PT.ET. Extracted Lycopene and Viscosity, Surface Tension Characterization by Man Singh Survismeter”,World Journal of Pharmaceutical Research, 2014,3, 778-784

- Bijendra Singh,V.K.Yadav,R.suriyaprabha,Samreen Heena Khan,G.Gananamoorty,Nisha Choudhary,Amit Kumar Yadav,Haresh Kalasariya, “A novel and efficient method for the synthesis of amorphous nanosilica from fly ash tiles” , Elsevier materials Today Proceedings, Volume 26, Part 2, 2020, Pages 701-705 [IF.065].

- Bijendra Singh,Virendra Kumar Yadav ,Nisha Chaudhary , “characterization of Indian Incense stick powder for their Physical ,Chemical and Mineralogical properties” World journal of Environmental Bioscience,Volume 9,Issue I:39-43.

- Singh,B.,Yadav,V.K.Kumar,P.,Kalasariya,H.Choudhary,N.Gananamoorthy,G.Gupta,N.Khan,S.H. & Khayal ,A.(2020). “The current Scenario of Indian Incense Market and Their Impact on the Indian Economy”, Ind.J.PureApp.Biosci.8(3),627-636.doi;http//dx.doi.org/10.18782/2582-2845.8168.

- Bijendra Singh ,Virendra Kumar Yadav, Nisha Choudhary, G. Gnanamoorthy, “The Positive and Negative Aspects of Incense Sticks Manufacturing, Burning and Disposal on Human and Environment”, World Journal of Environmental Biosciences, Vol. 9(2020).

- Bijendra Singh,Virendra K. Yadav,Pallavi Saxena,Chagan Lal,Govindhan Gnanamoorthy,Nisha Chaudhary,Neha Taveker,H.Kalasariya,Pankaj Kumar, “Synthesis and Characterization of Mullites froSilicoaluminous Fly Ash waste ” https://www.igi-global.com/journal/international-journal-applied-nanotechnology-research/214903

DOI: 10.4018/IJANR.20200101.oa2

Book Chapter

Bijendra Singh,Nisha Chaudhary,Virendra Kumar Yadav ,Parth Malik,Samreen Heena Khan,Gajendra Kumar Inwati,suriyaprabha R.,A.K. Yadav ,Raman Kumar Ravi, “Recovery of Natural Nanostructured Minerals: Ferrospheres,Plerospheres,Censpheres,and Carbonaceous Particles From fly Ash”, https://www.igi-global.com/book/handbook-research-emerging-developments-environmental/231905 ,Doi ; 10.4018/978-1-7998-1241-8.ch021.