Flexible screen-printed electrodes using stone paper as a substrate for electrochemical detection of cancer biomarker

Codruta Varodi, Florina Pogacean, Marin Gheorghe, Valentin Mirel, Maria Coros, and Stela Pruneanu

National Institute for Research and Development of Isotopic and Molecular Technologies, Donat Street, No. 67-103, RO- 400293, Cluj-Napoca, Romania

Abstract

Flexible electrodes printed on stone paper (HP) and modified with gold nanoparticles (HP-AuNPs) and graphene (HP-Gr-AuNPs) were manufactured. Their electrochemical properties were studied by linear sweep voltammetry (LSV) and electrochemical impedance spectroscopy (EIS). The obtained results indicate the beneficial effect of AuNPs and graphene: a low charge transfer resistance and a high apparent heterogeneous electron transfer rate, compared to the unmodified electrode (HP). In addition, the modified electrodes showed increased sensitivity towards 8-OHdG, such as 5.90 mA/M (for HP-AuNPs) and 9.29 mA/M (for HP-Gr-AuNPs), in comparison with the unmodified HP electrode (2.52 mA/M).

Keywords— biomolecules detection, graphene, screen-printed electrodes; 8-OHdG;

Professional Biography

Codruta Mihaela Varodi is Senior Researcher III at INCDTIM Cluj-Napoca. She received her B.Sc. in Chemical-Physics, Babes-Bolyai University, Cluj-Napoca, Romania (1994-1998) and her Ph.D. in Chemistry at Babes-Bolyai University, Cluj-Napoca, Romania (2016).

She has published more than 25 papers in ISI journals.

E-mail: codruta.varodi@itim-cj.ro

Acknowledgment

This work was supported by a grant of Ministry of Research and Innovation, CNCS-UEFISCDI, project number PN-III-P4-ID-PCCF-2016-0006, within PNCDI III and by Program Nucleu, no. PN 19 35 01 02/2019.