Abstract:

The study area is located in Red Sea basin which extend parallel to the Sudanese Red Sea in order to determine and estimate the source rocks in the basin and defined the sub-basins by using the geochemistry and satellite gravity method.

The Red Sea rift marks the break-up of the Afro-Arabian plate in Eocenee-Oligocene time. The tectonostratigraphic evolution can be subdivided into three major phases: Pre-rift, Syn-rift and Post-rift phase.

The geophysical studies which applied to the area by satellite gravity method. The Bouger anomaly, Residual anomaly and vertical derivative maps have been created. The basin appears in maps as low anomaly in between two high anomalies in East and West directions, this high anomaly interpreted as Mid Oceanic Ridge (MOR) and Red Sea Hills (RSH) respectively. By using the vertical derivative map, found the sub-basin, which contain certain vertical faults what bordered these sub-basins. Also about 8 sections done across the profiles in order to identified the accurate depths of basin where the wells were drilled, and it gave positive values.

With more than 12 wells being drilled obtained from RRI, some of which have been petroleum discoveries (gas and condensate with oil shows), Plots of Total Organic Carbon (TOC), Hydrogen Index (HI), Tmax, Oxygen Index (OI), Spore Color Index, Production Index and Paleo-tectonic sections were been created.

The results were found very good, according to the gravity method most of borehole have been located in shallow area. Very deep sub basins found in whole of the basin have been identified as good potential area to future drilling. Also, the formations of: (Kareem, Rudeis, Belayim, Dungunab, Zeit and Wardan) were classified as beasts’ source rocks in the drilled boreholes.

Finally, the study was concluded with highly potential to fined hydrocarbons in the study area. Then the recommendations were created in order to make it as nature when that is possible.