Simulation Study of Different Modes of Miscible Carbon Dioxide Flooding

Ehab Elakhal

Khalda Petroleum Company, Cairo, Egypt

Abstract

Carbone dioxide flooding has been applied worldwide as a successful enhanced oil recovery. Carbone dioxide flooding may be applied as a continuous injection or as water-alternating-gas (WAG) process. Optimization of the injection mode of carbon dioxide is important for economical field application. This paper focuses on using a fully compositional simulation model for “AEB-3C” sandstone oil reservoir of one of the Western Desert oil fields in Egypt to predict the impact of CO2 miscible flooding on the reservoir oil recovery and net present value (NPV), to define the best mode of operation that is straight CO2 injection or water alternating gas (WAG) processes and to show the difference between pure and impure CO2. Moreover, several sensitivity runs were done on the oil price to show minimum profitable value of oil price when applying such a tertiary method in the subject field. AEB-3C reservoir is a sandstone reservoir contains black oil of 38⁰ API and of 0.303 cp viscosity at reservoir temperature of 238⁰F. The reservoir depth at 10500 ft MD and average thickness of 40 ft with a porosity range of 9-19% and 112 md average permeability. The subject reservoir is penetrated by 11 holes. The sand is extended along the whole field as it isn’t lost in any of the drilled wells. The geologic model and rock typing were built based on the conventional core data taken in Well-A09. The PVT model representing the reservoir fluid was created based on the PVT laboratory tests done on the fluid sample taken from Well-A02ST.

The reservoir under study has been producing under a successful water flooding project since 
May-2010. The recovery factor by the end of water flooding project is predicted as 32%. 
CO2 flooding processes have started by the end of water flooding. A significant increase in the oil recovery factor was noticed due to applying this method; it reached up to 57%. Comparisons between different modes of operations were shown which showed better results when applying WAG process than that with straight CO2 injection. Moreover; sensitivities were done on the cycle periods in WAG processes and showed increase in the recovery factor with shortening the cycle periods. In addition to a comparison between pure and impure CO2 which showed very close results.

Biography

Ehab M. Elakhal has completed the M.Sc. in Reservoir Engineering in 2017 from Suez university. He got a Diploma in the Applied Geophysics from Cairo university in 2009. He got the B.Sc. of Petroleum Engineering from Suez university in 2007. He is currently a Section Head of Reservoir Engineering Dept. in Khalda Petroleum Company.

References

[1] Amaefule, J. O. (1993, Oct. 3-6). Enhanced Reservoir Description Using Core and Log Data to Identify Hydraulic Flow Units and Predict Permeability in Un-Cored Intervals/Wells. SPE Annual Technical Conference and Exhibition, Houston, Society of Petroleum Engineering 26436, pp. 1-16.

[2] Wyllie, M. a. (1950). Some Theoritical Considerations Related to the Quantitative Evaluation of the Physical Characteristics of Reservoir Rock from Electrical log Data. Journal Petroleum Technology.

[3] Uguru, C. I. (2005, August 1-3). Permeability Prediction Using Genetic Unit Averages of Flow Zone Indicators (FZIs) and Neural Networks. Society of Petroleum Engineers, SPE 98828, 29th Annual SPE International Technical Conference and Exhibition, pp. 1-8.

[4] Amaefule, J. O. (1993, Oct. 3-6). Enhanced Reservoir Description Using Core and Log Data to Identify Hydraulic Flow Units and Predict Permeability in Un-Cored Intervals/Wells. SPE Annual Technical Conference and Exhibition, Houston, Society of Petroleum Engineering 26436, pp. 1-16.

[5] Peng, D. a. (1976). A New-Constant Equation of State. Ind. & Eng. Chem., 15, No. 1, 59.

[6] Ahmed, T. (2010). Reservoir Engineering Handbook-4th Edition. Burlington,USA: ELSEVIER Inc.

[7] Skjaeveland, S. e. (2000, February). Capillary Pressure Correlation for Mixed-Wet Reservoirs. SPE Reservoir Eval. & Eng., 3, pp. 1-2.

[8] CARTER, R. D., & TRACY, G. W. (1960, August 30). An Improved Method for Calculating Water Influx. SPE 1626-G, 219, pp. 1-3.

[9] Alston, R. K. (1985, Apr.). CO2 Minimum Miscibility Pressure: A Correlation for Impure CO2 Streams and Live Oil Systems. SPEJ, pp. 298-274.

[10] Zhiqiang, G. a. (2009). Applicability of carbondioxide enhanced oil recovery to reservoirs in the unita basin, utah. Utah: Utah department of natural resources.