CHARACTERIZATION OF COMPONENTS ISOLATED FROM
ALGERIAN APRICOT SHELLS (PRUNUS ARMENIACA L.)
DJENNAT ALLOUCH,* MARCEL POPA,**,*** VALENTIN I. POPA,** GABRIELA LISA,** ADRIAN CĂTĂLIN PUIȚEL** and HICHEM NASRI*

*Laboratory of Biodiversity and Pollution of Ecosystems, Faculty of Nature and Life Sciences, University
Chadli Bendjedid El Tarf, El Tarf 36000, Algeria
**Gheorghe Asachi Technical University, Faculty of Chemical Engineering and Environmental Protection
“Cristofor Simionescu”, 73, Prof.dr.doc. D. Mangeron Blvd., 700050 Iaşi, Romania
***Academy of Romanian Scientists, 54, Splaiul Independentei, 050085 Bucuresti, Romania
✉Corresponding author: Djennat Allouch, espoirenfance@hotmail.fr

Shells resulting from food processing and   agricultural activities, such as hard shells of apricot, are considered as wastes and are generally used as fuel. However, this residue shows promise as lignocellulosic feedstock for biorefineries, for its conversion to liquid fuel or bio-products. This study is dedicated to the characterization and isolation of lignin, cellulose and hemicelluloses from apricot shells (AS). The chemical composition and thermal stability of AS after chemical treatment with solvents (ethanol-toluene), cellulose, hemicelluloses and lignin were analyzed by standard methods, i.e. Fourier-transform infrared spectroscopy (FTIR) and thermogravimetric analysis (TGA). Further, almost 50.2 wt% of a water-insoluble extract was obtained after the bleaching process, which showed the removal of lignin, and the final percentages for this extraction were 50.2 ± 0.34%, 26.5 ± 0.83%, 23.7 ± 0.29% and 35 ± 1% for holocellulose, cellulose, hemicelluloses and lignin, respectively. FTIR spectroscopy evidenced the structure of lignin, cellulose and hemicelluloses. Thermal analysis and the kinetic study suggested that cellulose had higher thermal stability than the other components, with the activation energy of 289.62 kJ/mol. Thus, our results indicated the high potential of AS to be used as an environmentally friendly material in a biorefinery, as well as in the modern polymer and chemical industries.

Keywords: apricot shells, lignocellulosic, chemical composition, holocellulose, cellulose, hemicelluloses, lignin, kinetic study, thermogravimetric analysis (TGA)

Recent Publications

Djennat allouch,Marcel popa,Valentin i. popa,Gabriela lisa, Adrian cătălin puițeland Hichem nasri,characterization of components isolated from algerian apricot shells (prunus armeniaca l.), Cellulose Chem. Technol., 53 (9-10), 851-859(2019).

Biography

teacher assistant professor in the faculty of natural sciences and life of the university of El Oued, Algeria